Sinclair ZX Spectrum Emulator User Guide

ZX Spectrum 16K/48K: startup options and command console reference

Ignacio Cea Forniés

2026

# Contents

Contents	1

/o	6

/p	6

/h	6

/f	7

/k	7

/c	7

/l	8

/a	8

/d	8

/s	9

/i	9

/r	9

/g	10

/n	10

/w	10

/b	11

/m	11

/h	12

/f	12

/k	12

/c	13

/l	13

/a	13

/d	14

/s	14

/i	14

/r	15

/p	15

/g	15

/n	16

/w	16

/b	16

/m	17

3.1 Standard Framework Commands	18

HELP	18

AUTHOR	18

STATUS	18

PC	19

REGISTERS	19

CHANGEREG	19

STACK	20

CPUSTATUS	20

CPUSSTATUS	20

CPUSTATE	21

CPUINFO	21

MEMORY	21

MEMORYDUMP	22

MEMORYSTR	22

SETMEMORY	22

MEMORYANALYSIS	23

STOP	23

RUN	24

SETPC	24

NEXT	24

NEXTSTACK	25

SHOWNEXT	25

INST	25

BREAKS	26

SETBREAK	26

REMOVEBREAK	26

REMOVEBREAKS	27

SPEED	27

LOADBIN	27

SAVEBIN	28

DEBUGON	28

DEBUGOFF	28

IDEBUGON	29

IDEBUGOFF	29

RESTART	29

DEVICES	30

PERIPHERALS	30

PERCOMMAND	30

ASSIGNJOY	31

CLOCKFACTOR	31

SOUNDON	31

SOUNDOFF	32

INTERRUPTS	32

INTSET	32

CHIPS	33

CRTON	33

CRTOFF	33

DATASETTE	34

GRIDON	34

GRIDOFF	34

PICTURE	35

SETHOOK	35

REMOVEHOOK	35

HOOKS	36

HOOKSHELP	36

CMOVEPARAMS	36

3.2 Inherited Sinclair Commands	37

SYSVARS	37

SYSVAR	37

3.3 ZX Spectrum-Specific Commands	38

ULA	38

ULAEVENTS	38

SCREENDUMP	38

COLORDUMP	39

CHARSDRAW	39

3.4 Local-Console-Only Commands	39

LOADPRG	39

LOADBINARY	40

LOADBLOCKS	40

DECOMPILE	40

POSSIBLEPERS	41

CONNECTPER	41

DISCONNECTPERS	42

LOADPERDATA	42

EMPTYPERDATA	42

SAVEPERDATA	43

CLEARPERDATA	43

Appendix A. Command Availability Summary	44

Appendix B. Source-of-Truth Maintenance Rule	44

Illustrations	45

Equations	46

Foreword

This guide describes the startup and command interfaces implemented by the two Sinclair ZX Spectrum executables in EMULATORS: ZXSpectrumEmulator (without a local command console) and ZXSpectrumEmulatorC (with a local command console). It covers the standard 16K/48K machine, PAL and NTSC video, four ROM languages, and is based on both entry points, the ZXSPECTRUM::ZXSpectrumEmulator inheritance chain, LocalConsole, and the complete ZXSPECTRUM -> SINCLAIR -> StandardCommandBuilder responsibility chain.

Command and parameter names are shown exactly as accepted by the current code. Startup options use /XVALUE with no space between the one-letter option and its value. Command names are matched as written by the builders; use uppercase names for reliable operation. Positional command parameters are separated by spaces; examples use paths without embedded spaces.

The non-console executable does not expose an interactive prompt. When started with /o it creates a remote communication system using the same ZX Spectrum, Sinclair and standard command builders. Commands marked Local console only are intercepted by LocalConsole and therefore cannot be sent to the non-console executable.

1. Startup Parameters: Version Without a Local Console

Executable: ZXSpectrumEmulator.exe. It accepts the framework and ZX Spectrum-specific options below, plus /o and the communication-port option. Option letters and language values are case-sensitive because the command-line parser preserves them exactly.

# /o

Meaning: Controls creation of the remote command communication system.

General form:

/o[YES|NO]

- YES or an empty value: open remote communications.

- NO (and any other non-YES non-empty value): do not open it.

- Omitted: remote communications disabled.

Examples:

ZXSpectrumEmulator.exe /o

ZXSpectrumEmulator.exe /oYES /p60000

Consequence: When enabled, remote clients can execute ZX Spectrum, Sinclair and standard builder commands. LocalConsole-only commands remain unavailable.

# /p

Meaning: Selects the TCP listening port used by the non-console executable's remote command channel.

General form:

/pPORTNUMBER

- 0 through 65535: requested listening port; values above 1000 are recommended.

- Omitted: port 60000 is used when /o enables the channel.

- Collision: the inherited emulator also interprets /p as a comma-separated peripheral-ID list. A supplied port number is therefore also attempted as a peripheral ID during initialization.

Examples:

ZXSpectrumEmulator.exe /o

ZXSpectrumEmulator.exe /o /p61000

Consequence: The remote server listens on the selected port. When /p is present, the same value is also attempted as a peripheral ID and normally produces a log warning.

# /h

Meaning: Prints the executable help and prevents the emulation from starting.

General form:

/h

- No value.

Examples:

ZXSpectrumEmulator.exe /h

Consequence: Initialization returns failure after printing the help; the current main program therefore exits with code 1.

# /f

Meaning: Loads a binary file whose header contains the destination address.

General form:

/fFILE

- FILE: path to the header-addressed binary file; attach it directly to /f.

Examples:

ZXSpectrumEmulator.exe /fprogram.bin

ZXSpectrumEmulator.exe /fgames\demo.bin /a$080d

Consequence: The file is loaded before execution. An unreadable or invalid file aborts initialization.

# /k

Meaning: Loads a block-structured memory file.

General form:

/kFILE

- FILE: path to a block file understood by the framework.

Examples:

ZXSpectrumEmulator.exe /ksnapshot.blocks

Consequence: Every encoded block is placed at its stored address. A load error aborts initialization.

# /c

Meaning: Parses, assembles and loads an assembler source file.

General form:

/cFILE

- FILE: assembler source path.

Examples:

ZXSpectrumEmulator.exe /cstartup.asm /s

Consequence: The assembled bytes are installed before the main loop. Syntax or load errors are logged and initialization stops.

# /l

Meaning: Selects the framework diagnostic level.

General form:

/lLEVEL

- 0: no diagnostics.

- 1: errors (default).

- 2: dump at exit.

- 3: errors and warnings.

- 4: all regular diagnostics.

- 5: internal trace diagnostics.

Examples:

ZXSpectrumEmulator.exe /l0

ZXSpectrumEmulator.exe /l4

Consequence: Higher values produce progressively more diagnostic output and may reduce performance.

# /a

Meaning: Overrides the address at which execution begins.

General form:

/aADDRESS

- ADDRESS: ZX Spectrum address in the framework number syntax, for example decimal or hexadecimal with $.

Examples:

ZXSpectrumEmulator.exe /a$080d

ZXSpectrumEmulator.exe /fprogram.bin /a2061

Consequence: A non-zero, valid address replaces the normal reset/program entry address.

# /d

Meaning: Creates startup stop points at one or more addresses.

General form:

/dADDRESS1,ADDRESS2,...

- ADDRESSn: comma-separated address. Each entry accepts the framework number syntax.

Examples:

ZXSpectrumEmulator.exe /d$080d

ZXSpectrumEmulator.exe /d$080d,$ffd2 /s

Consequence: Execution stops when the CPU reaches any listed address, allowing later inspection or continuation.

# /s

Meaning: Controls whether the emulated computer starts stopped.

General form:

/s[YES|NO]

- YES or an empty value: start stopped.

- NO (and any non-YES non-empty value): start running.

- Omitted: start running.

Examples:

ZXSpectrumEmulator.exe /s

ZXSpectrumEmulator.exe /sNO

Consequence: When enabled, the machine is initialized but the CPU does not run until a RUN/NEXT command or the equivalent UI action is issued.

# /i

Meaning: Selects the ZX Spectrum ROM language.

General form:

/iLANG

- ENG: English (default).

- ESP: Spanish.

- NOR: Nordic.

- ARA: Arabic.

- Language identifiers are case-sensitive; use the uppercase forms above. Any other value loads the English ROM.

Examples:

ZXSpectrumEmulator.exe /iESP

ZXSpectrumEmulator.exe /iARA

Consequence: The selected 16 KiB 48K-family ROM image is loaded before initialization; the screen's glyph reference remains sourced from the English ROM file.

# /r

Meaning: Selects the initial ULA sound-output state.

General form:

/r[YES|NO]

- YES or an empty value: sound enabled.

- NO (and any other non-YES non-empty value): sound disabled.

- Omitted: sound enabled.

Examples:

ZXSpectrumEmulator.exe /r

ZXSpectrumEmulator.exe /rNO

Consequence: The ZX Spectrum sound device attached to the ULA sound function starts enabled or disabled as requested.

# /g

Meaning: Controls the CRT post-processing effect at startup.

General form:

/g[YES|NO]

- YES or an empty value: CRT effect enabled.

- NO (and any non-YES non-empty value): disabled.

- Omitted: disabled.

Examples:

ZXSpectrumEmulator.exe /g

ZXSpectrumEmulator.exe /gNO

Consequence: The screen starts with or without CRT emulation. CRTON and CRTOFF can change it later.

# /n

Meaning: Selects NTSC timing and screen geometry instead of PAL.

General form:

/n

- No value: the presence of the option selects NTSC; omission selects PAL.

Examples:

ZXSpectrumEmulator.exe /n

Consequence: The factory creates the NTSC ULA, NTSC screen and 3.5275 MHz clock. Without /n it creates the PAL ULA, PAL screen and 3.5 MHz clock.

# /w

Meaning: Selects the installed RAM configuration.

General form:

/wCONF

- 0 or omitted: 16 KiB RAM configuration (default).

- 1: 48 KiB RAM configuration.

- 2 or any value greater than 1: unsupported. The current range check can pass or clamp it to the invalid internal value 2, which may assert in debug builds and must not be used.

Examples:

ZXSpectrumEmulator.exe /w0

ZXSpectrumEmulator.exe /w1

Consequence: Configuration 0 maps 16 KiB RAM with unmapped upper memory; configuration 1 maps the full 48 KiB RAM area before ROM startup.

# /b

Meaning: Draws the diagnostic character-cell grid over the active screen and selects its startup colour.

General form:

/b[COLOR]

- COLOR omitted or 0: colour 0 (default).

- 1 through 3: the requested startup grid colour.

- Any value greater than 3: clamped to 3 by the startup parser.

Examples:

ZXSpectrumEmulator.exe /b

ZXSpectrumEmulator.exe /b3

Consequence: The screen starts with its diagnostic 8x8 grid enabled. The later GRIDON console command has a wider 0-15 renderer range.

# /m

Meaning: Selects the ZX Spectrum machine family implemented by this emulator.

General form:

/mMACHINE

- ZXSTD: standard 16K/48K ZX Spectrum; this is also the default.

- Any other value: silently falls back to ZXSTD in the current implementation.

Examples:

ZXSpectrumEmulator.exe /mZXSTD

ZXSpectrumEmulator.exe /mZXSTD /w1 /iESP

Consequence: Only the original standard ZX Spectrum type is created. This selector does not provide 128K, +2, +2A or +3 models; /w, /i and /n independently select RAM, ROM language and video standard.

2. Startup Parameters: Version With a Local Console

Executable: ZXSpectrumEmulatorC.exe. Its main program delegates the complete command line to ZXSPECTRUM::ZXSpectrumEmulator, so all inherited framework options and ZX Spectrum-specific options are active even though the short main-level banner only advertises /h. It has no /o listener; commands are entered in its local console.

# /h

Meaning: Prints the executable help and prevents the emulation from starting.

General form:

/h

- No value.

Examples:

ZXSpectrumEmulatorC.exe /h

Consequence: Initialization returns failure after printing the help; the current main program therefore exits with code 1.

# /f

Meaning: Loads a binary file whose header contains the destination address.

General form:

/fFILE

- FILE: path to the header-addressed binary file; attach it directly to /f.

Examples:

ZXSpectrumEmulatorC.exe /fprogram.bin

ZXSpectrumEmulatorC.exe /fgames\demo.bin /a$080d

Consequence: The file is loaded before execution. An unreadable or invalid file aborts initialization.

# /k

Meaning: Loads a block-structured memory file.

General form:

/kFILE

- FILE: path to a block file understood by the framework.

Examples:

ZXSpectrumEmulatorC.exe /ksnapshot.blocks

Consequence: Every encoded block is placed at its stored address. A load error aborts initialization.

# /c

Meaning: Parses, assembles and loads an assembler source file.

General form:

/cFILE

- FILE: assembler source path.

Examples:

ZXSpectrumEmulatorC.exe /cstartup.asm /s

Consequence: The assembled bytes are installed before the main loop. Syntax or load errors are logged and initialization stops.

# /l

Meaning: Selects the framework diagnostic level.

General form:

/lLEVEL

- 0: no diagnostics.

- 1: errors (default).

- 2: dump at exit.

- 3: errors and warnings.

- 4: all regular diagnostics.

- 5: internal trace diagnostics.

Examples:

ZXSpectrumEmulatorC.exe /l0

ZXSpectrumEmulatorC.exe /l4

Consequence: Higher values produce progressively more diagnostic output and may reduce performance.

# /a

Meaning: Overrides the address at which execution begins.

General form:

/aADDRESS

- ADDRESS: ZX Spectrum address in the framework number syntax, for example decimal or hexadecimal with $.

Examples:

ZXSpectrumEmulatorC.exe /a$080d

ZXSpectrumEmulatorC.exe /fprogram.bin /a2061

Consequence: A non-zero, valid address replaces the normal reset/program entry address.

# /d

Meaning: Creates startup stop points at one or more addresses.

General form:

/dADDRESS1,ADDRESS2,...

- ADDRESSn: comma-separated address. Each entry accepts the framework number syntax.

Examples:

ZXSpectrumEmulatorC.exe /d$080d

ZXSpectrumEmulatorC.exe /d$080d,$ffd2 /s

Consequence: Execution stops when the CPU reaches any listed address, allowing later inspection or continuation.

# /s

Meaning: Controls whether the emulated computer starts stopped.

General form:

/s[YES|NO]

- YES or an empty value: start stopped.

- NO (and any non-YES non-empty value): start running.

- Omitted: start running.

Examples:

ZXSpectrumEmulatorC.exe /s

ZXSpectrumEmulatorC.exe /sNO

Consequence: When enabled, the machine is initialized but the CPU does not run until a RUN/NEXT command or the equivalent UI action is issued.

# /i

Meaning: Selects the ZX Spectrum ROM language.

General form:

/iLANG

- ENG: English (default).

- ESP: Spanish.

- NOR: Nordic.

- ARA: Arabic.

- Language identifiers are case-sensitive; use the uppercase forms above. Any other value loads the English ROM.

Examples:

ZXSpectrumEmulatorC.exe /iESP

ZXSpectrumEmulatorC.exe /iARA

Consequence: The selected 16 KiB 48K-family ROM image is loaded before initialization; the screen's glyph reference remains sourced from the English ROM file.

# /r

Meaning: Selects the initial ULA sound-output state.

General form:

/r[YES|NO]

- YES or an empty value: sound enabled.

- NO (and any other non-YES non-empty value): sound disabled.

- Omitted: sound enabled.

Examples:

ZXSpectrumEmulatorC.exe /r

ZXSpectrumEmulatorC.exe /rNO

Consequence: The ZX Spectrum sound device attached to the ULA sound function starts enabled or disabled as requested.

# /p

Meaning: Connects one or more peripherals during startup.

General form:

/pPER1[,PER2...]

- 10: typewriter keyboard input.

- 100: Sinclair cassette signal simulation.

- 101: ZX Spectrum TAP/TZX cassette-data injection.

- 102: ZX Printer thermal-printer simulation.

- 201: cartridge implementation supported by the builder, although it is not listed by POSSIBLEPERS.

Examples:

ZXSpectrumEmulatorC.exe /p10

ZXSpectrumEmulatorC.exe /p100,102

Consequence: Each requested peripheral is built and connected to the first compatible device during initialization. Unsupported IDs are logged and initialization continues.

# /g

Meaning: Controls the CRT post-processing effect at startup.

General form:

/g[YES|NO]

- YES or an empty value: CRT effect enabled.

- NO (and any non-YES non-empty value): disabled.

- Omitted: disabled.

Examples:

ZXSpectrumEmulatorC.exe /g

ZXSpectrumEmulatorC.exe /gNO

Consequence: The screen starts with or without CRT emulation. CRTON and CRTOFF can change it later.

# /n

Meaning: Selects NTSC timing and screen geometry instead of PAL.

General form:

/n

- No value: the presence of the option selects NTSC; omission selects PAL.

Examples:

ZXSpectrumEmulatorC.exe /n

Consequence: The factory creates the NTSC ULA, NTSC screen and 3.5275 MHz clock. Without /n it creates the PAL ULA, PAL screen and 3.5 MHz clock.

# /w

Meaning: Selects the installed RAM configuration.

General form:

/wCONF

- 0 or omitted: 16 KiB RAM configuration (default).

- 1: 48 KiB RAM configuration.

- 2 or any value greater than 1: unsupported. The current range check can pass or clamp it to the invalid internal value 2, which may assert in debug builds and must not be used.

Examples:

ZXSpectrumEmulatorC.exe /w0

ZXSpectrumEmulatorC.exe /w1

Consequence: Configuration 0 maps 16 KiB RAM with unmapped upper memory; configuration 1 maps the full 48 KiB RAM area before ROM startup.

# /b

Meaning: Draws the diagnostic character-cell grid over the active screen and selects its startup colour.

General form:

/b[COLOR]

- COLOR omitted or 0: colour 0 (default).

- 1 through 3: the requested startup grid colour.

- Any value greater than 3: clamped to 3 by the startup parser.

Examples:

ZXSpectrumEmulatorC.exe /b

ZXSpectrumEmulatorC.exe /b3

Consequence: The screen starts with its diagnostic 8x8 grid enabled. The later GRIDON console command has a wider 0-15 renderer range.

# /m

Meaning: Selects the ZX Spectrum machine family implemented by this emulator.

General form:

/mMACHINE

- ZXSTD: standard 16K/48K ZX Spectrum; this is also the default.

- Any other value: silently falls back to ZXSTD in the current implementation.

Examples:

ZXSpectrumEmulatorC.exe /mZXSTD

ZXSpectrumEmulatorC.exe /mZXSTD /w1 /iESP

Consequence: Only the original standard ZX Spectrum type is created. This selector does not provide 128K, +2, +2A or +3 models; /w, /i and /n independently select RAM, ROM language and video standard.

3. Command Console Reference

The local console checks its eleven emulator-level commands first. All remaining text is delegated through ZXSPECTRUM::CommandBuilder, then SINCLAIR::CommandBuilder, and finally MCHEmul::StandardCommandBuilder. The remote /o channel in ZXSpectrumEmulator.exe uses only that three-builder chain. The availability line in each entry makes this boundary explicit.

# 3.1 Standard Framework Commands

## HELP

Availability: Local console and remote /o channel.

Accepted aliases: CHELP.

Meaning: Displays the ZX Spectrum console help assembled from the ZX Spectrum, Sinclair, local-console and standard help files.

General syntax:

HELP

Parameters:

- No parameters.

Examples:

HELP

Result and consequences: Displays the ZX Spectrum console help assembled from the ZX Spectrum, Sinclair, local-console and standard help files.

## AUTHOR

Availability: Local console and remote /o channel.

Accepted aliases: CAUTHOR.

Meaning: Returns framework author information.

General syntax:

AUTHOR

Parameters:

- No parameters.

Examples:

AUTHOR

Result and consequences: Returns framework author information.

## STATUS

Availability: Local console and remote /o channel.

Accepted aliases: CSTATUS.

Meaning: Shows the Z80 status register.

General syntax:

STATUS

Parameters:

- No parameters.

Examples:

STATUS

Result and consequences: Shows the Z80 status register.

## PC

Availability: Local console and remote /o channel.

Accepted aliases: CPC.

Meaning: Shows the current program counter.

General syntax:

PC

Parameters:

- No parameters.

Examples:

PC

Result and consequences: Shows the current program counter.

## REGISTERS

Availability: Local console and remote /o channel.

Accepted aliases: CREGISTERS.

Meaning: Shows all CPU registers.

General syntax:

REGISTERS

Parameters:

- No parameters.

Examples:

REGISTERS

Result and consequences: Shows all CPU registers.

## CHANGEREG

Availability: Local console and remote /o channel.

Accepted aliases: CCHANGEREG.

Meaning: Changes one CPU internal register.

General syntax:

CHANGEREG REGNAME VALUE

Parameters:

- REGNAME: registered Z80 register name.

- VALUE: new decimal, z-prefixed binary, o-prefixed octal or $-prefixed hexadecimal value.

Examples:

CHANGEREG A $41

Result and consequences: The register changes only if it exists and accepts the supplied byte width; otherwise an error is returned.

## STACK

Availability: Local console and remote /o channel.

Accepted aliases: CSTACK.

Meaning: Shows stack state.

General syntax:

STACK [ALL]

Parameters:

- ALL: optional; includes stack contents in addition to basic state.

Examples:

STACK

STACK ALL

Result and consequences: Shows stack state.

## CPUSTATUS

Availability: Local console and remote /o channel.

Accepted aliases: CCPUSTATUS.

Meaning: Shows registers, status, program counter and stack.

General syntax:

CPUSTATUS

Parameters:

- No parameters.

Examples:

CPUSTATUS

Result and consequences: Shows registers, status, program counter and stack.

## CPUSSTATUS

Availability: Local console and remote /o channel.

Accepted aliases: CCPUSSTATUS.

Meaning: Shows the compact CPU status without stack detail.

General syntax:

CPUSSTATUS

Parameters:

- No parameters.

Examples:

CPUSSTATUS

Result and consequences: Shows the compact CPU status without stack detail.

## CPUSTATE

Availability: Local console and remote /o channel.

Accepted aliases: CCPUSTATE.

Meaning: Shows the CPU execution state and state-specific data.

General syntax:

CPUSTATE

Parameters:

- No parameters.

Examples:

CPUSTATE

Result and consequences: Shows the CPU execution state and state-specific data.

## CPUINFO

Availability: Local console and remote /o channel.

Accepted aliases: CCPUINFO.

Meaning: Returns the complete CPU information structure.

General syntax:

CPUINFO

Parameters:

- No parameters.

Examples:

CPUINFO

Result and consequences: Returns the complete CPU information structure.

## MEMORY

Availability: Local console and remote /o channel.

Accepted aliases: CMEMORY.

Meaning: Reads the active CPU-visible memory range.

General syntax:

MEMORY INITIAL [FINAL]

Parameters:

- INITIAL: first address.

- FINAL: optional last address; with no final address only the initial location is reported.

Examples:

MEMORY $0400

MEMORY $0400 $0427

Result and consequences: Reads the active CPU-visible memory range.

## MEMORYDUMP

Availability: Local console and remote /o channel.

Accepted aliases: CMEMORYDUMP.

Meaning: Dumps every memory view between two addresses, including inactive RAM/ROM views.

General syntax:

MEMORYDUMP INITIAL FINAL

Parameters:

- INITIAL: first address.

- FINAL: last address.

Examples:

MEMORYDUMP $a000 $a03f

Result and consequences: Dumps every memory view between two addresses, including inactive RAM/ROM views.

## MEMORYSTR

Availability: Local console and remote /o channel.

Accepted aliases: CMEMORYSTR.

Meaning: Shows the configured memory structure.

General syntax:

MEMORYSTR [ALL]

Parameters:

- ALL: optional; includes inactive regions. Without it only active regions are shown.

Examples:

MEMORYSTR

MEMORYSTR ALL

Result and consequences: Shows the configured memory structure.

## SETMEMORY

Availability: Local console and remote /o channel.

Accepted aliases: CSETMEMORY.

Meaning: Writes a value to one address or repeatedly across a range.

General syntax:

SETMEMORY INITIAL [FINAL] VALUE

Parameters:

- INITIAL: first address.

- FINAL: optional last address.

- VALUE: byte sequence in framework numeric syntax; multi-byte values advance by their own length.

Examples:

SETMEMORY $0400 $41

SETMEMORY $0400 $0427 $20

Result and consequences: The active memory view is modified immediately; unsafe writes can corrupt state or crash the emulated program.

## MEMORYANALYSIS

Availability: Local console and remote /o channel.

Accepted aliases: CMEMORYANALYSIS.

Meaning: Analyzes the current memory configuration.

General syntax:

MEMORYANALYSIS

Parameters:

- No parameters.

Examples:

MEMORYANALYSIS

Result and consequences: Analyzes the current memory configuration.

## STOP

Availability: Local console and remote /o channel.

Accepted aliases: CSTOP.

Meaning: Stops CPU execution.

General syntax:

STOP

Parameters:

- No parameters.

Examples:

STOP

Result and consequences: Stops CPU execution.

## RUN

Availability: Local console and remote /o channel.

Accepted aliases: R, CRUN.

Meaning: Starts or resumes CPU execution; alias R.

General syntax:

RUN [ADDRESS]

Parameters:

- ADDRESS: optional new program counter before running.

Examples:

RUN

R $080d

Result and consequences: The CPU resumes immediately, optionally after changing PC.

## SETPC

Availability: Local console and remote /o channel.

Accepted aliases: CSETPC.

Meaning: Sets the program counter without starting execution.

General syntax:

SETPC ADDRESS

Parameters:

- ADDRESS: new CPU address.

Examples:

SETPC $080d

Result and consequences: Subsequent execution continues from the new address; an unsafe address can break the program.

## NEXT

Availability: Local console and remote /o channel.

Accepted aliases: N, CNEXT.

Meaning: Executes the next instruction; alias N.

General syntax:

NEXT

Parameters:

- No parameters.

Examples:

NEXT

N

Result and consequences: Executes the next instruction; alias N.

## NEXTSTACK

Availability: Local console and remote /o channel.

Accepted aliases: NS, CNEXTSTACK.

Meaning: Runs until the stack pointer returns to its initial value; alias NS.

General syntax:

NEXTSTACK

Parameters:

- No parameters.

Examples:

NS

Result and consequences: Useful for stepping over a routine, although behavior depends on the routine's stack discipline.

## SHOWNEXT

Availability: Local console and remote /o channel.

Accepted aliases: SN, CSHOWNEXT.

Meaning: Disassembles upcoming instruction(s); alias SN.

General syntax:

SHOWNEXT [NUMBER]

Parameters:

- NUMBER: optional count; one instruction by default.

Examples:

SHOWNEXT

SN 8

Result and consequences: Disassembles upcoming instruction(s); alias SN.

## INST

Availability: Local console and remote /o channel.

Accepted aliases: CINST.

Meaning: Shows the last instruction executed.

General syntax:

INST

Parameters:

- No parameters.

Examples:

INST

Result and consequences: Shows the last instruction executed.

## BREAKS

Availability: Local console and remote /o channel.

Accepted aliases: CBREAKS.

Meaning: Lists all stop-point addresses.

General syntax:

BREAKS

Parameters:

- No parameters.

Examples:

BREAKS

Result and consequences: Lists all stop-point addresses.

## SETBREAK

Availability: Local console and remote /o channel.

Accepted aliases: CSETBREAK.

Meaning: Adds one or more breakpoints.

General syntax:

SETBREAK ADDRESS1 [ADDRESS2 ...]

Parameters:

- ADDRESSn: one or more addresses in framework numeric syntax.

Examples:

SETBREAK $080d $ffd2

Result and consequences: Adds one or more breakpoints.

## REMOVEBREAK

Availability: Local console and remote /o channel.

Accepted aliases: CREMOVEBREAK.

Meaning: Removes selected breakpoints.

General syntax:

REMOVEBREAK ADDRESS1 [ADDRESS2 ...]

Parameters:

- ADDRESSn: one or more existing breakpoint addresses.

Examples:

REMOVEBREAK $ffd2

Result and consequences: Removes selected breakpoints.

## REMOVEBREAKS

Availability: Local console and remote /o channel.

Accepted aliases: CREMOVEBREAKS.

Meaning: Removes every breakpoint.

General syntax:

REMOVEBREAKS

Parameters:

- No parameters.

Examples:

REMOVEBREAKS

Result and consequences: Removes every breakpoint.

## SPEED

Availability: Local console and remote /o channel.

Accepted aliases: CSPEED.

Meaning: Shows measured CPU and screen-I/O execution speed.

General syntax:

SPEED

Parameters:

- No parameters.

Examples:

SPEED

Result and consequences: Shows measured CPU and screen-I/O execution speed.

## LOADBIN

Availability: Local console and remote /o channel.

Accepted aliases: CLOADBIN.

Meaning: Loads a raw binary file at an explicit address.

General syntax:

LOADBIN FILE ADDRESS

Parameters:

- FILE: raw binary path.

- ADDRESS: first destination address.

Examples:

LOADBIN charset.bin $2000

Result and consequences: Bytes are copied into memory without an embedded load-address header.

## SAVEBIN

Availability: Local console and remote /o channel.

Accepted aliases: CSAVEBIN.

Meaning: Saves a memory range as raw binary.

General syntax:

SAVEBIN FILE SIZE ADDRESS

Parameters:

- FILE: output path.

- SIZE: byte count.

- ADDRESS: first source address.

Examples:

SAVEBIN screen.bin 1000 $0400

Result and consequences: Saves a memory range as raw binary.

## DEBUGON

Availability: Local console and remote /o channel.

Accepted aliases: CACTIVATEDEEPDEBUG.

Meaning: Enables deep execution debugging.

General syntax:

DEBUGON FILE CPU [APPEND] [FILTERS...]

Parameters:

- FILE: output log.

- CPU: YES includes CPU execution; NO omits it.

- APPEND: optional YES appends, NO overwrites (default).

- D:START-END or ALLDIR: address filter.

- C:IDs or ALLCHIP: chip filter.

- I:IDs or ALLIO: device filter.

- M:IDs or ALLMEM: memory filter.

Examples:

DEBUGON zxspectrum.log YES NO D:$0800-$0fff ALLCHIP ALLIO ALLMEM

Result and consequences: Tracing begins with the selected sources and can substantially slow emulation. It cannot coexist with interrupt-only debugging.

## DEBUGOFF

Availability: Local console and remote /o channel.

Accepted aliases: CDESACTIVATEDEEPDEBUG.

Meaning: Stops deep execution debugging.

General syntax:

DEBUGOFF

Parameters:

- No parameters.

Examples:

DEBUGOFF

Result and consequences: Stops deep execution debugging.

## IDEBUGON

Availability: Local console and remote /o channel.

Accepted aliases: CIDEBUGON.

Meaning: Enables interrupt-only debugging.

General syntax:

IDEBUGON FILE [APPEND]

Parameters:

- FILE: output log.

- APPEND: optional YES appends; NO overwrites (default).

Examples:

IDEBUGON interrupts.log NO

Result and consequences: Interrupt tracing begins and cannot coexist with deep debugging.

## IDEBUGOFF

Availability: Local console and remote /o channel.

Accepted aliases: CIDEBUGOFF.

Meaning: Stops interrupt-only debugging.

General syntax:

IDEBUGOFF

Parameters:

- No parameters.

Examples:

IDEBUGOFF

Result and consequences: Stops interrupt-only debugging.

## RESTART

Availability: Local console and remote /o channel.

Accepted aliases: CRESTART.

Meaning: Requests a restart and exits the current computer loop so it can be rebuilt.

General syntax:

RESTART STOPPED [LEVEL]

Parameters:

- STOPPED: YES restarts stopped; NO restarts running.

- LEVEL: optional 0 CPU-only/mild restart (default), 1 computer restart preserving memory, greater than 1 full restart.

Examples:

RESTART YES 0

RESTART NO 2

Result and consequences: Requests a restart and exits the current computer loop so it can be rebuilt.

## DEVICES

Availability: Local console and remote /o channel.

Accepted aliases: CDEVICES.

Meaning: Shows all or selected I/O devices.

General syntax:

DEVICES [ID1 ID2 ...]

Parameters:

- IDn: optional device identifiers; with none, all devices are returned.

Examples:

DEVICES

DEVICES 1 2

Result and consequences: Shows all or selected I/O devices.

## PERIPHERALS

Availability: Local console and remote /o channel.

Accepted aliases: CPERIPHERALS.

Meaning: Shows all or selected connected peripherals.

General syntax:

PERIPHERALS [ID1 ID2 ...]

Parameters:

- IDn: optional peripheral identifiers; with none, all are returned.

Examples:

PERIPHERALS

PERIPHERALS 100

Result and consequences: Shows all or selected connected peripherals.

## PERCOMMAND

Availability: Local console and remote /o channel.

Accepted aliases: CPERCMD.

Meaning: Sends a device-specific command to a connected peripheral.

General syntax:

PERCOMMAND PERID CMDID [ATTRS...]

Parameters:

- PERID: connected peripheral ID.

- CMDID: command ID implemented by that peripheral.

- ATTRS: optional positional attributes forwarded unchanged.

Examples:

PERCOMMAND 100 0

Result and consequences: The peripheral executes the request if both IDs and its parameter contract are valid; otherwise the answer reports an error.

## ASSIGNJOY

Availability: Local console and remote /o channel.

Accepted aliases: CASSIGNJ.

Meaning: Remaps a joystick identifier.

General syntax:

ASSIGNJOY ID NEWID

Parameters:

- ID: current joystick ID.

- NEWID: required target ID; using the same ID removes an existing assignment.

Examples:

ASSIGNJOY 0 1

Result and consequences: Remaps a joystick identifier.

## CLOCKFACTOR

Availability: Local console and remote /o channel.

Accepted aliases: CCLOCKFACTOR.

Meaning: Changes the CPU clock multiplier.

General syntax:

CLOCKFACTOR VALUE

Parameters:

- VALUE: numeric factor; 1 means normal speed.

Examples:

CLOCKFACTOR 2

Result and consequences: The emulation pacing target changes; values above one request faster execution.

## SOUNDON

Availability: Local console and remote /o channel.

Accepted aliases: CSOUNDON.

Meaning: Enables ZX Spectrum ULA sound output.

General syntax:

SOUNDON

Parameters:

- No parameters.

Examples:

SOUNDON

Result and consequences: Enables the sound system attached to the ULA sound function.

## SOUNDOFF

Availability: Local console and remote /o channel.

Accepted aliases: CSOUNDOFF.

Meaning: Disables ZX Spectrum ULA sound output.

General syntax:

SOUNDOFF

Parameters:

- No parameters.

Examples:

SOUNDOFF

Result and consequences: Disables the sound system; emulation continues without audible ULA output.

## INTERRUPTS

Availability: Local console and remote /o channel.

Accepted aliases: CINTERRUPTS.

Meaning: Shows all interrupts known by the computer.

General syntax:

INTERRUPTS

Parameters:

- No parameters.

Examples:

INTERRUPTS

Result and consequences: Shows all interrupts known by the computer.

## INTSET

Availability: Local console and remote /o channel.

Accepted aliases: CINTSET.

Meaning: Enables or disables one interrupt or every interrupt.

General syntax:

INTSET ID|ALL ON|OFF

Parameters:

- ID or ALL: target interrupt.

- ON or OFF: requested state.

Examples:

INTSET ALL ON

Result and consequences: An unknown interrupt ID produces an error.

## CHIPS

Availability: Local console and remote /o channel.

Accepted aliases: CCHIPS.

Meaning: Lists the emulated chips and their identifying attributes.

General syntax:

CHIPS

Parameters:

- No parameters.

Examples:

CHIPS

Result and consequences: Lists the emulated chips and their identifying attributes.

## CRTON

Availability: Local console and remote /o channel.

Accepted aliases: CCRTON.

Meaning: Enables the CRT screen effect.

General syntax:

CRTON

Parameters:

- No parameters.

Examples:

CRTON

Result and consequences: Enables the CRT screen effect.

## CRTOFF

Availability: Local console and remote /o channel.

Accepted aliases: CCRTOFF.

Meaning: Disables the CRT screen effect.

General syntax:

CRTOFF

Parameters:

- No parameters.

Examples:

CRTOFF

Result and consequences: Disables the CRT screen effect.

## DATASETTE

Availability: Local console and remote /o channel.

Accepted aliases: CDATASETTE.

Meaning: Shows datasette state when a supported datasette is connected.

General syntax:

DATASETTE

Parameters:

- No parameters.

Examples:

DATASETTE

Result and consequences: Shows datasette state when a supported datasette is connected.

## GRIDON

Availability: Local console and remote /o channel.

Accepted aliases: CGRIDON.

Meaning: Enables the diagnostic grid overlay on the screen.

General syntax:

GRIDON COLOR

Parameters:

- COLOR: mandatory numeric grid color.

Examples:

GRIDON 14

Result and consequences: Enables the standard screen grid with the requested color.

## GRIDOFF

Availability: Local console and remote /o channel.

Accepted aliases: CGRIDOFF.

Meaning: Disables the diagnostic grid overlay.

General syntax:

GRIDOFF

Parameters:

- No parameters.

Examples:

GRIDOFF

Result and consequences: Disables the standard screen grid.

## PICTURE

Availability: Local console and remote /o channel.

Accepted aliases: CPICTURE.

Meaning: Saves a screenshot of the emulator screen.

General syntax:

PICTURE FILE

Parameters:

- FILE: output PNG, JPG or BMP filename.

Examples:

PICTURE zxspectrum-screen.png

Result and consequences: A supported extension writes the current screen; unsupported formats fail.

## SETHOOK

Availability: Local console and remote /o channel.

Accepted aliases: CSETHOOK.

Meaning: Creates an execution hook.

General syntax:

SETHOOK ID TYPE [PARAMS...]

Parameters:

- ID: unique hook identifier.

- TYPE: hook kind; currently 0 stops on memory read/write.

- For TYPE 0, PARAMS are mandatory INITIAL and optional FINAL addresses.

Examples:

SETHOOK 1 0 $d000 $d3ff

Result and consequences: Creates an execution hook.

## REMOVEHOOK

Availability: Local console and remote /o channel.

Accepted aliases: CREMOVEHOOK.

Meaning: Removes a hook by identifier.

General syntax:

REMOVEHOOK ID

Parameters:

- ID: hook identifier.

Examples:

REMOVEHOOK 1

Result and consequences: If the hook does not exist, no state changes.

## HOOKS

Availability: Local console and remote /o channel.

Accepted aliases: CHOOKS.

Meaning: Lists active hooks.

General syntax:

HOOKS

Parameters:

- No parameters.

Examples:

HOOKS

Result and consequences: Lists active hooks.

## HOOKSHELP

Availability: Local console and remote /o channel.

Accepted aliases: CHOOKSHELP.

Meaning: Shows help for supported hook types.

General syntax:

HOOKSHELP

Parameters:

- No parameters.

Examples:

HOOKSHELP

Result and consequences: Shows help for supported hook types.

## CMOVEPARAMS

Availability: Local console and remote /o channel.

Accepted aliases: No public alias.

Meaning: Builds the framework's internal MoveParametersToAnswerCommand.

General syntax:

CMOVEPARAMS

Parameters:

- No parameters are executable under the current command contract.

Examples:

CMOVEPARAMS

Result and consequences: The command returns an empty answer when invoked directly; it is an internal response helper and is not advertised in the help file.

# 3.2 Inherited Sinclair Commands

## SYSVARS

Availability: Local console and remote /o channel.

Accepted aliases: CSYSVARS.

Meaning: Returns every system variable defined for the selected ZX Spectrum ROM profile.

General syntax:

SYSVARS

Parameters:

- No parameters.

Examples:

SYSVARS

Result and consequences: Returns the current values using the system-variable definitions loaded for the selected ROM language.

## SYSVAR

Availability: Local console and remote /o channel.

Accepted aliases: CSYSVAR.

Meaning: Returns one named ZX Spectrum system variable.

General syntax:

SYSVAR VNAME

Parameters:

- VNAME: mandatory variable name from the active ZX Spectrum system-variable table.

Examples:

SYSVAR FRAMES

SYSVAR RAMTOP

Result and consequences: Returns the named variable and its current memory value; an unknown name returns an error entry.

# 3.3 ZX Spectrum-Specific Commands

## ULA

Availability: Local console and remote /o channel.

Accepted aliases: CULA.

Meaning: Returns the current ZX Spectrum ULA and video-generation state.

General syntax:

ULA

Parameters:

- No parameters.

Examples:

ULA

Result and consequences: Returns the active ULA state, including raster and display-generation information for PAL or NTSC operation.

## ULAEVENTS

Availability: Local console and remote /o channel.

Accepted aliases: CULAEVENTS.

Meaning: Enables or disables visualization of significant CPU/ULA events.

General syntax:

ULAEVENTS ON|OFF

Parameters:

- ON: enables event visualization.

- OFF: disables it. The implementation accepts any single token; only the exact value ON enables the feature.

Examples:

ULAEVENTS ON

ULAEVENTS OFF

Result and consequences: Changes the event-overlay flag used by the active ZX Spectrum ULA.

## SCREENDUMP

Availability: Local console and remote /o channel.

Accepted aliases: CSCREENDUMP.

Meaning: Returns a snapshot of ZX Spectrum bitmap memory.

General syntax:

SCREENDUMP

Parameters:

- No parameters.

Examples:

SCREENDUMP

Result and consequences: Returns the current 6,144-byte bitmap area selected by the machine's screen-memory snapshot routine.

## COLORDUMP

Availability: Local console and remote /o channel.

Accepted aliases: CCOLORDUMP.

Meaning: Returns a snapshot of ZX Spectrum attribute memory.

General syntax:

COLORDUMP

Parameters:

- No parameters.

Examples:

COLORDUMP

Result and consequences: Returns the current 768-byte colour-attribute area selected by the machine's colour-memory snapshot routine.

## CHARSDRAW

Availability: Local console and remote /o channel.

Accepted aliases: CCHARSDRAW.

Meaning: Returns textual drawings of all or selected ZX Spectrum character glyphs.

General syntax:

CHARSDRAW [0..95 ...]

Parameters:

- Character indexes: optional values from 0 through 95; out-of-range values are ignored and duplicates are coalesced. With none, all 96 glyphs are rendered.

Examples:

CHARSDRAW

CHARSDRAW 0 31 64 95

Result and consequences: Returns textual 8x8 glyph drawings from the standard English 48K ROM character data used by the screen object.

# 3.4 Local-Console-Only Commands

## LOADPRG

Availability: Local console only.

Accepted aliases: None.

Meaning: Loads, assembles and installs an assembler source through the emulator object.

General syntax:

LOADPRG FILE

Parameters:

- FILE: assembler source path.

Examples:

LOADPRG demo.asm

Result and consequences: Compilation or loading errors are reported by the local console.

## LOADBINARY

Availability: Local console only.

Accepted aliases: None.

Meaning: Loads a binary containing its destination address in the file header.

General syntax:

LOADBINARY FILE

Parameters:

- FILE: header-addressed binary path.

Examples:

LOADBINARY demo.bin

Result and consequences: Loads a binary containing its destination address in the file header.

## LOADBLOCKS

Availability: Local console only.

Accepted aliases: None.

Meaning: Loads a block-structured memory file.

General syntax:

LOADBLOCKS FILE

Parameters:

- FILE: block file path.

Examples:

LOADBLOCKS state.blocks

Result and consequences: Loads a block-structured memory file.

## DECOMPILE

Availability: Local console only.

Accepted aliases: None.

Meaning: Disassembles a memory region.

General syntax:

DECOMPILE ADDRESS BYTES

Parameters:

- ADDRESS: first address.

- BYTES: approximate byte count; output rounds to complete instructions.

Examples:

DECOMPILE $080d 64

Result and consequences: Disassembles a memory region.

## POSSIBLEPERS

Availability: Local console only.

Accepted aliases: None.

Meaning: Lists the peripheral IDs and construction attributes advertised by the ZX Spectrum I/O peripheral builder.

General syntax:

POSSIBLEPERS

Parameters:

- No parameters.

Examples:

POSSIBLEPERS

Result and consequences: Lists IDs 10 (typewriter), 100 (cassette), 101 (cassette injection) and 102 (thermal printer). Cartridge ID 201 is accepted by the builder but is not advertised.

## CONNECTPER

Availability: Local console only.

Accepted aliases: None.

Meaning: Builds and connects a peripheral to the first compatible ZX Spectrum device.

General syntax:

CONNECTPER ID [ATTRS...]

Parameters:

- ID: peripheral-emulation ID (10, 100, 101, 102 or the additionally supported cartridge ID 201).

- ATTRS: optional construction attributes. For printer 102, use f:FILENAME and p:THERMAL or p:THERMAL-PS.

Examples:

CONNECTPER 100

CONNECTPER 102 f:printer.ps p:THERMAL-PS

Result and consequences: Connection succeeds only when the ID, attributes and a compatible target device are available; otherwise the answer reports an error.

## DISCONNECTPERS

Availability: Local console only.

Accepted aliases: None.

Meaning: Disconnects one or more peripherals.

General syntax:

DISCONNECTPERS ID1 [ID2 ...]

Parameters:

- IDn: connected peripheral IDs.

Examples:

DISCONNECTPERS 100 101

Result and consequences: Disconnects one or more peripherals.

## LOADPERDATA

Availability: Local console only.

Accepted aliases: None.

Meaning: Loads media or program data into a connected ZX Spectrum peripheral.

General syntax:

LOADPERDATA ID FILE [DEBUGFILE]

Parameters:

- ID: connected peripheral ID.

- FILE: supported data file; for example sampled TZX data for ID 100, or TAP/TAP-like TZX data for injection ID 101.

- DEBUGFILE: optional file that starts full-range format-related debugging after a successful load.

Examples:

LOADPERDATA 100 tape.tzx

LOADPERDATA 101 game.tap load-trace.log

Result and consequences: The operation fails if the peripheral rejects the file type. Injection accepts TAP and only TZX files whose blocks are all TAP-compatible.

## EMPTYPERDATA

Availability: Local console only.

Accepted aliases: None.

Meaning: Creates and attaches empty media data when the peripheral implements that operation.

General syntax:

EMPTYPERDATA ID FILE

Parameters:

- ID: connected peripheral ID.

- FILE: destination whose extension selects a writable format.

Examples:

EMPTYPERDATA 100 blank.tzx

Result and consequences: The Sinclair cassette can create an empty TZX container; unsupported peripherals or formats return an error.

## SAVEPERDATA

Availability: Local console only.

Accepted aliases: None.

Meaning: Saves a peripheral's current media data.

General syntax:

SAVEPERDATA ID [FILE]

Parameters:

- ID: connected peripheral ID.

- FILE: optional output path; it may be omitted when a prior load/create operation established the filename.

Examples:

SAVEPERDATA 100

SAVEPERDATA 100 copy.tzx

Result and consequences: Saves current cassette data when the peripheral can retrieve it and the selected format is writable. Injection ID 101 cannot retrieve data.

## CLEARPERDATA

Availability: Local console only.

Accepted aliases: None.

Meaning: Clears all data currently loaded into peripherals.

General syntax:

CLEARPERDATA

Parameters:

- No parameters.

Examples:

CLEARPERDATA

Result and consequences: Commonly used to force a subsequent program/media reload.

Appendices

# Appendix A. Command Availability Summary

The table distinguishes the interactive console from the command channel of the non-console executable. Remote availability assumes ZXSpectrumEmulator.exe was started with /o and that the client is connected.

| Command family | ZXSpectrumEmulatorC local console | ZXSpectrumEmulator remote /o |
| --- | --- | --- |
| Standard builder | Yes | Yes |
| Inherited Sinclair builder | Yes | Yes |
| ZX Spectrum builder | Yes | Yes |
| LocalConsole emulator/media operations | Yes | No |

# Appendix B. Source-of-Truth Maintenance Rule

This file is a maintained repository artifact. Every change that adds, removes, renames or changes a ZX Spectrum startup option, command-builder registration, LocalConsole-only command, syntax, parameter meaning, consequence or visible formatted output must include a review and, when applicable, an update of docs/ZXSpectrumData/ZXSpectrumEmulator_UserGuide.docx. Update the corresponding canonical .fmt source whenever the command's InfoStructure contract changes.

Audit the complete chain, not only the edited class: both ZXSpectrumEmulator entry points and the ZXSPECTRUM::ZXSpectrumEmulator hierarchy for startup options; LocalConsole for local-only commands; and ZXSPECTRUM::CommandBuilder -> SINCLAIR::CommandBuilder -> MCHEmul::StandardCommandBuilder for builder commands.

# Illustrations

No se encuentran elementos de tabla de ilustraciones.NO ILLUSTRATIONS ARE INCLUDED IN THIS EDITION.

# Equations

No se encuentran elementos de tabla de ilustraciones.NO EQUATIONS ARE INCLUDED IN THIS EDITION.