C264 Family DEBUG File

EMULATORS deep-debug file format

Ignacio Cea Forniés

2026

# Contents

Contents	1

Foreword	3

1. Purpose, scope, and models	4

1.1 Scope of this guide	4

2. Activation and producer selection	5

2.1 Console command	5

2.2 Conditions and special paths	5

3. Common file structure	6

3.1 Physical hierarchy	6

3.2 Timing rules	6

4. Common framework records	7

CPU — Info Cycle	7

CPU — Transaction about to execute	7

CPU — Executed instruction	7

CPU — Stop	7

CPU — Interrupts	8

Memory — Applied Set	8

5. Machine-specific components	9

5.1 Selectable inventory	9

5.2 Record catalog	9

TED — Info Cycle	9

TED — CPU transaction projection	9

ROMRAMSwitch — direct line	9

C1C2Selector — direct line	10

Datasette 1531 / SerialIO	10

6. Interpretation method	11

6.1 Recommended procedure	11

6.2 Annotated example	11

7. Limitations and maintenance	12

7.1 Specific limitations	12

7.2 Maintenance contract	12

Appendices	13

Appendix A. Provenance and revision	13

A.1 Watched source files	14

A.2 Revision history	14

Illustrations	14

Equations	15

Foreword

This guide defines the DEBUG format for C264 Family. Its purpose is to make a trace readable and auditable directly against the code that produces it. Audience: Framework and emulator developers, timing-test authors, and users diagnosing execution traces.

1. Purpose, scope, and models

Provide a verifiable description of the format, provenance, timing, and interpretation of EMULATORS deep-debug files.

# 1.1 Scope of this guide

Covered models: Commodore 16, Commodore 116, and Commodore Plus/4, PAL or NTSC.

Main processor: MOS 7501/8501, implemented through C7501.

The guide describes the behavior implemented in the current repository revision. When a component is present but its producer contains a TODO, the guide explicitly states that it currently produces no output.

2. Activation and producer selection

# 2.1 Console command

DEBUGON FILENAME YES|NO [YES|NO] [D:DIR1-DIR2] [ALLCHIP|C:id ...] [ALLIO|I:id ...] [ALLMEM|M:id ...]
DEBUGOFF
IDEBUGON FILENAME [YES|NO]
IDEBUGOFF

| Parameter | Meaning |
| --- | --- |
| FILENAME | Output file path. |
| First YES¦NO | YES connects the CPU to DEBUG; NO omits CPU records. |
| Second YES¦NO | YES appends to the file; NO creates or truncates it. The default is NO. |
| D:start-end | Restricts CPU records to the stated PC range. It does not by itself filter selected chips. |
| ALLCHIP / C:id | Selects every chip or specific identifiers. |
| ALLIO / I:id | Selects every device or specific identifiers. |
| ALLMEM / M:id | Selects every memory unit or specific identifiers. |

# 2.2 Conditions and special paths

- DEBUGON and IDEBUGON are mutually exclusive.

- Deactivation is deferred when requested while an instruction is executing, so that a block is not cut in half.

- The canonical internal command is CACTIVATEDEEPDEBUG; DEBUGON is its console alias. DEBUGOFF is an alias for CDESACTIVATEDEEPDEBUG.

- An additional path exists: when peripheral data is loaded with a third parameter, the console can activate an appended CPU trace without explicitly selecting chips, devices, or memory.

3. Common file structure

# 3.1 Physical hierarchy

| Element | Representation |
| --- | --- |
| Complete header | COMPONENT + absolute cycle + record class. Component and cycle are fitted into 8- and 12-character columns. |
| Subordinate data | Five tabs of indentation. An attribute uses a name fitted to 20 characters, a colon, and its value. |
| Simple text | Written without columns or indentation; it is used, for example, in the IDEBUGON preamble. |
| Direct write | Some older producers build tab-separated lines with operator<< and do not guarantee exactly the same column widths. |
| Block separator | A line containing only + closes one complete computer iteration. It is emitted after CPU, chip, and I/O processing while DEBUG is active. |

# 3.2 Timing rules

- The header cycle is the emulator's absolute counter unless the machine-specific section documents an anomaly or different unit.

- In complete-instruction mode, the CPU finishes the instruction before chips recover their cycles. Membership of a block does not prove the exact physical cycle of a mapped write.

- Chip snapshots are normally written before the recovered cycle is processed. A subordinate event occurs during that cycle, and its resulting state is usually visible in the next snapshot.

- Additional Info records do not count as ordinary cycles when checking instruction duration.

- Duration comparisons must be restarted around stop blocks or interrupt entry.

- An isolated PC jump does not prove an interrupt; explicit evidence or reconstruction from the vector and stack is required.

4. Common framework records

# CPU — Info Cycle

Producer: src/CORE/CPU.cpp :: CPU::debugLastExecutionData / debugInstructionWaiting

Condition: At the start of a CPU iteration, or while waiting for an instruction in cycle-by-cycle mode.

Fields: Absolute cycle; Last N cycles, or Instruction in N cycles.

Interpretation: Last N cycles belongs to the previous instruction. The following PC line describes the instruction that has just executed in the current block. Duration is compared between consecutive blocks.

Status: Implemented

# CPU — Transaction about to execute

Producer: src/CORE/CPU.cpp :: CPU::debugInstructionAboutToExecute / debugInterruptAboutToExecute

Condition: Before consuming an instruction transaction or launching an interrupt.

Fields: Transaction(Type, Address/Id, Code, Cycles); Bus cycle data(Defined, Structure, Reads, Writes, TrailingWrites, LastCycleType); Execution(Mode, State, LastState).

Interpretation: Exposes the selected nominal bus structure. LastCycleType uses CPUCycle flags: read=2 and write=4; interpret other values according to CPUTransaction.hpp.

Status: Implemented

# CPU — Executed instruction

Producer: src/CORE/CPU.cpp :: CPU::debugInstructionExecuted

Condition: After a complete instruction has executed successfully.

Fields: Previous PC and stack position; disassembly; IN/OUT address and data when present; State/Last State; status register; CPU registers.

Interpretation: The parenthesised annotation is the effective address and the data observed or transferred by the instruction, not a separate new record.

Status: Implemented

# CPU — Stop

Producer: src/CORE/CPU.cpp :: debugStopRequest / debugStopSituation / debugAlreadyStopped

Condition: Stop request, entry into the stopped state, and later cycles still covered by the stop.

Fields: Serialised StopStatusData; State; LastState; number of pending cycles.

Interpretation: Stop request is the request; Stopped describes the applied state; Already stopped confirms that the validity counter remains active.

Status: Implemented

# CPU — Interrupts

Producer: src/CORE/CPU.cpp and src/CORE/CPUInterrupt.cpp

Condition: Request, wait, rejection, launch, failure, or start of interrupt code.

Fields: Serialised request; Id; Priority; LaunchCycles; bus structure; Clock and PC.

Interpretation: Repeated requests while a line remains active do not prove repeated edges. Correlate them with wait, not allowed, launched, and CPUINT records.

Status: Implemented

# Memory — Applied Set

Producer: src/CORE/Memory.cpp :: SetMemoryCommand::execute

Condition: Application of a memory write stored as a command.

Fields: Subset name; cycle shown as -; hexadecimal Address; hexadecimal Value.

Interpretation: The dash means that the producer supplies no absolute cycle. The write is documented when the command is applied to the subset.

Status: Implemented

5. Machine-specific components

# 5.1 Selectable inventory

| Id | Component | DEBUG output |
| --- | --- | --- |
| 101 | TED | Raster, single/double clock, graphics modes, memory, badlines, and integrated sound |
| 200/201 | 6529B | Keyboard/model latches; they currently have no dedicated DEBUG producer |
| 202 | ROM/RAM switch | ROM Active state on FF3E/FF3F access |
| 203 | C1/C2 selector | Selected memory configuration at FDD0/FDDF |
| I/O | Datasette 1531/IEC | Tape traps and serial peripherals |
| Memory | C264 map | Applied Set for selected subsets |

# 5.2 Record catalog

## TED — Info Cycle

Producer: src/COMMODORE/TED/TED.cpp :: debugTEDCycle

Fields: Info Cycle type with In Single Clock indicator; Raster position; Instruction projection; Graphics mode; Memory(Screen, Attributes, Characters and ROM used, Bitmap).

Timing: The cycle column currently receives i, not the absolute CPU counter. Treat it as the recovered-loop index.

Events: Bad line situation; Reading Video Matrix & Attribute RAM; Reading Graphics.

## TED — CPU transaction projection

Producer: src/COMMODORE/TED/TED.cpp :: debugCPUAboutToExecute

Fields: Transaction(Type, Address, Code, CPU Cycles); raster origin; PositionsToEffect; SingleClock.

Timing: Emitted when instruction context is received; it relates CPU cycles to TED positions.

Events: Adds no subordinate events.

## ROMRAMSwitch — direct line

Producer: src/C264/ROMRAMSwitch.cpp :: debugROMRAMSwitchCycle

Fields: Component FF3E/FF3F,X; CPU cycle; Info cycle; ROM Active:YES|NO.

Timing: Emitted only when configurationChanged() is true.

Events: Legacy tab-separated format.

## C1C2Selector — direct line

Producer: src/C264/C1C2Selector.cpp :: debugC1C2SelectorCycle

Fields: Component FDD0/FDDF,X; CPU cycle; Info cycle; Memory Cfg:number.

Timing: Emitted only when the configuration changes.

Events: Legacy tab-separated format.

## Datasette 1531 / SerialIO

Producer: src/C264/1531Datasette.cpp and src/COMMODORE/SerialIOPeripherals.cpp

Fields: Header/data found, block, simulation state, error or missing data; serial-bus transitions.

Timing: CPU cycle of the trap or transition.

Events: May mix subordinate text and complete headers.

6. Interpretation method

# 6.1 Recommended procedure

- Identify the block delimited by + and note the absolute CPU cycle.

- Read Transaction about to execute and the PC line to establish the instruction, mode, and nominal bus structure.

- Count each chip's ordinary cycles, excluding Additional Info and projection records.

- Correlate subordinate events with the snapshot they follow and check their effect in the next cycle.

- For interrupts, build the sequence request → wait/rejection → launch → CPUINT → effective entry.

- Record observed facts, inferences, and timing limitations separately.

# 6.2 Annotated example

CPU     50000       Info Cycle
                    Last 3 cycles
TED     0           Info Cycle (In Single Clock:YES)
                    Raster position     :Column=...,Row=...,25
                    Instruction projection:PositionsToEffect=...,SingleClock=1
FF3E/FF3F,X    50000    Info cycle      ROM Active:YES
+

Schematic example. The TED time field is i and must not be read as an absolute CPU cycle.

7. Limitations and maintenance

# 7.1 Specific limitations

- The debugTEDCycle header uses i, so the absolute-continuity rules used for other chips cannot be applied directly.

- The 6529B latches are part of the machine but produce no DEBUG lines of their own.

- ROM/RAM and C1/C2 retain a legacy tab-separated format with variable-length labels.

# 7.2 Maintenance contract

- The current code is the semantic authority, not an old LOG file or help text.

- Every change to a DEBUG producer requires review of debug-common.yaml or the relevant machine source and regeneration of the DOCX files.

- check_debug_docs.py compares the structured sources, watched C++ files, and generated artefacts with generated-state.json.

- Items marked TODO exist in the emulator assembly but currently write no fields; they are documented to avoid promising output that does not exist.

Appendices

# Appendix A. Provenance and revision

Structured sources: .codex/documentation/debug-files/sources/debug-common.yaml and .codex/documentation/debug-files/sources/c264.yaml.

# A.1 Watched source files

| Path | Scope |
| --- | --- |
| include/CORE/DebugFile.hpp | Common |
| src/CORE/DebugFile.cpp | Common |
| src/CORE/Computer.cpp | Common |
| src/CORE/CPU.cpp | Common |
| src/CORE/CPUInterrupt.cpp | Common |
| src/CORE/Memory.cpp | Common |
| src/CORE/StdCommands.cpp | Common |
| src/CONSOLE/LocalConsole.cpp | Common |
| src/C264/C264.cpp | c264 |
| src/C264/C1C2Selector.cpp | c264 |
| src/C264/ROMRAMSwitch.cpp | c264 |
| src/C264/1531Datasette.cpp | c264 |
| src/COMMODORE/TED/TED.cpp | c264 |
| src/COMMODORE/SerialIOPeripherals.cpp | c264 |

# A.2 Revision history

| Version | Year | Description |
| --- | --- | --- |
| 1.0 | 2026 | Initial inventory generated and checked against the current C++ producers. |

# Illustrations

No table of illustrations entries were found.

# Equations

No table of illustrations entries were found.