Commodore 64 DEBUG File

Structure, fields, and interpretation criteria

Ignacio Cea Forniés

2026

# Contents

Actualice los campos para generar el índice.

Foreword

This guide defines the DEBUG format for Commodore 64. Its purpose is to make a trace readable and auditable directly against the code that produces it. Audience: Framework and emulator developers, timing-test authors, and users diagnosing execution traces.

1. Purpose, scope, and models

Provide a verifiable description of the format, provenance, timing, and interpretation of EMULATORS deep-debug files.

# 1.1 Scope of this guide

Covered models: Commodore 64 PAL and NTSC; console and console-free executables.

Main processor: MOS 6510 (6500 family).

The guide describes the behavior implemented in the current repository revision. When a component is present but its producer contains a TODO, the guide explicitly states that it currently produces no output.

2. Activation and producer selection

# 2.1 Console command

DEBUGON FILENAME YES|NO [YES|NO] [D:DIR1-DIR2] [ALLCHIP|C:id ...] [ALLIO|I:id ...] [ALLMEM|M:id ...]
DEBUGOFF
IDEBUGON FILENAME [YES|NO]
IDEBUGOFF

| Parameter | Meaning |
| --- | --- |
| FILENAME | Output file path. |
| First YES¦NO | YES connects the CPU to DEBUG; NO omits CPU records. |
| Second YES¦NO | YES appends to the file; NO creates or truncates it. The default is NO. |
| D:start-end | Restricts CPU records to the stated PC range. It does not by itself filter selected chips. |
| ALLCHIP / C:id | Selects every chip or specific identifiers. |
| ALLIO / I:id | Selects every device or specific identifiers. |
| ALLMEM / M:id | Selects every memory unit or specific identifiers. |

# 2.2 Conditions and special paths

- DEBUGON and IDEBUGON are mutually exclusive.

- Deactivation is deferred when requested while an instruction is executing, so that a block is not cut in half.

- The canonical internal command is CACTIVATEDEEPDEBUG; DEBUGON is its console alias. DEBUGOFF is an alias for CDESACTIVATEDEEPDEBUG.

- An additional path exists: when peripheral data is loaded with a third parameter, the console can activate an appended CPU trace without explicitly selecting chips, devices, or memory.

3. Common file structure

# 3.1 Physical hierarchy

| Element | Representation |
| --- | --- |
| Complete header | COMPONENT + absolute cycle + record class. Component and cycle are fitted into 8- and 12-character columns. |
| Subordinate data | Five tabs of indentation. An attribute uses a name fitted to 20 characters, a colon, and its value. |
| Simple text | Written without columns or indentation; it is used, for example, in the IDEBUGON preamble. |
| Direct write | Some older producers build tab-separated lines with operator<< and do not guarantee exactly the same column widths. |
| Block separator | A line containing only + closes one complete computer iteration. It is emitted after CPU, chip, and I/O processing while DEBUG is active. |

# 3.2 Timing rules

- The header cycle is the emulator's absolute counter unless the machine-specific section documents an anomaly or different unit.

- In complete-instruction mode, the CPU finishes the instruction before chips recover their cycles. Membership of a block does not prove the exact physical cycle of a mapped write.

- Chip snapshots are normally written before the recovered cycle is processed. A subordinate event occurs during that cycle, and its resulting state is usually visible in the next snapshot.

- Additional Info records do not count as ordinary cycles when checking instruction duration.

- Duration comparisons must be restarted around stop blocks or interrupt entry.

- An isolated PC jump does not prove an interrupt; explicit evidence or reconstruction from the vector and stack is required.

4. Common framework records

# CPU — Info Cycle

Producer: src/CORE/CPU.cpp :: CPU::debugLastExecutionData / debugInstructionWaiting

Condition: At the start of a CPU iteration, or while waiting for an instruction in cycle-by-cycle mode.

Fields: Absolute cycle; Last N cycles, or Instruction in N cycles.

Interpretation: Last N cycles belongs to the previous instruction. The following PC line describes the instruction that has just executed in the current block. Duration is compared between consecutive blocks.

Status: Implemented

# CPU — Transaction about to execute

Producer: src/CORE/CPU.cpp :: CPU::debugInstructionAboutToExecute / debugInterruptAboutToExecute

Condition: Before consuming an instruction transaction or launching an interrupt.

Fields: Transaction(Type, Address/Id, Code, Cycles); Bus cycle data(Defined, Structure, Reads, Writes, TrailingWrites, LastCycleType); Execution(Mode, State, LastState).

Interpretation: Exposes the selected nominal bus structure. LastCycleType uses CPUCycle flags: read=2 and write=4; interpret other values according to CPUTransaction.hpp.

Status: Implemented

# CPU — Executed instruction

Producer: src/CORE/CPU.cpp :: CPU::debugInstructionExecuted

Condition: After a complete instruction has executed successfully.

Fields: Previous PC and stack position; disassembly; IN/OUT address and data when present; State/Last State; status register; CPU registers.

Interpretation: The parenthesised annotation is the effective address and the data observed or transferred by the instruction, not a separate new record.

Status: Implemented

# CPU — Stop

Producer: src/CORE/CPU.cpp :: debugStopRequest / debugStopSituation / debugAlreadyStopped

Condition: Stop request, entry into the stopped state, and later cycles still covered by the stop.

Fields: Serialised StopStatusData; State; LastState; number of pending cycles.

Interpretation: Stop request is the request; Stopped describes the applied state; Already stopped confirms that the validity counter remains active.

Status: Implemented

# CPU — Interrupts

Producer: src/CORE/CPU.cpp and src/CORE/CPUInterrupt.cpp

Condition: Request, wait, rejection, launch, failure, or start of interrupt code.

Fields: Serialised request; Id; Priority; LaunchCycles; bus structure; Clock and PC.

Interpretation: Repeated requests while a line remains active do not prove repeated edges. Correlate them with wait, not allowed, launched, and CPUINT records.

Status: Implemented

# Memory — Applied Set

Producer: src/CORE/Memory.cpp :: SetMemoryCommand::execute

Condition: Application of a memory write stored as a command.

Fields: Subset name; cycle shown as -; hexadecimal Address; hexadecimal Value.

Interpretation: The dash means that the producer supplies no absolute cycle. The write is documented when the command is applied to the subset.

Status: Implemented

5. Machine-specific components

# 5.1 Selectable inventory

| Id | Component | DEBUG output |
| --- | --- | --- |
| 200 | CIA1 | CIA cycle and keyboard/joystick Additional Info |
| 201 | CIA2 | CIA cycle; ports, timers, and connected signals |
| 203 | PLA | LORAM, HIRAM, CHAREN, GAME, and EXROM when configuration changes |
| 104 | VIC-II | Raster, graphics state, badlines, sprites, BA/AEC, and stop prediction |
| 105 | SID | Three-voice state, ADSR, and waveforms |
| Memory | C64 map | Configuration changes and Applied Set writes |
| I/O | Datasette/IEC/peripherals | Tape traps and serial-bus state when selected |

# 5.2 Record catalog

## VICII — Info Cycle

Producer: src/COMMODORE/VICII/VICII.cpp :: debugVICIICycle

Fields: Raster position; Internal(IDLE, VCBASE, VC, VLMI, GAccessIndex, RC, ROW, last data, BadlineCAccessStartedFromIdle, and complete badline state); CPU bus ownership(BA, AEC, windows, DMA masks, BadLineSource, and SpriteSourceMask); CPU stop prediction; Sprite DMA state(EnabledMask, ActiveMask, DisplayMask, YExpansionMask, YExpansionFlipFlopMask, and eight-entry Y, MCBASE, and MC lists); Border(Main, Vertical, and LastEventAt); Graphics mode; Memory location. List entries use sprite order 0..7 and | as separator. LastEventAt is the visible position of the latest horizontal-comparator transition of either border flip-flop.

Timing: Snapshot taken before the main processing of the recovered VIC-II cycle. The header uses cpu.clockCycles()-i. Horizontal border comparators evaluate physical raster pixels 0..3 before the CPU-phase register write and pixels 4..7 after it; the final-cycle vertical comparison observes the post-write state. Comparator results are translated to the aligned ScreenMemory slice, so a transition beyond its last pixel becomes visible in the following slice.

Events: In shared graphics cycles the g-access event precedes the c-access event; an optional CPU-phase register-write record follows all VIC-II bus activity. BadlineCAccessStartedFromIdle records the sequencer state latched at the start of the recognition cycle: its phi1 g-access remains idle while the phi2 c-access can occur in that same cycle. Sprite DMA [Sprite,Active,Y,MCBASE,MC,YExpansion,YExpansionFlipFlop] marks the exact cycle where a sprite DMA channel starts or stops. Horizontal border transitions are retained per aligned output pixel for later composition while the hardware flip-flops advance at the physical comparator. The vertical-border rules are evaluated at the exact left comparator and again at the final raster cycle. Sprite composition and pixel drawing occur later in the same absolute cycle.

## VICII — Video Matrix / graphics pipeline / pixel composition

Producer: src/COMMODORE/VICII/VICII.cpp :: debugReadingVideoMatrix, debugReadingGraphics, debugDrawPixelAt and debugDrawSpriteAt

Fields: Reading Video Matrix & Color RAM: Screen, Color, VLMI, VC, and Invalid. Reading Graphics: Address, Data, Screen, Color, GAccessIndex, VLMI, VC, RC, Idle, Mode, CharacterBase, and BitmapBase. Address is the exact VIC-visible address used by the completed memory read; Screen and Color are the values aligned with Data in the graphics fetch buffer. Drawing pixels: Raster, FirstPixel, LastPixel, XScroll, OutputPixel, ShiftActive, ShiftPixel, OutputData/Screen/Color, PendingReload, PendingData/Screen/Color, StagedAccess, StagedData/Screen/Color, ReloadInSpan, and ReloadPixel. FirstPixel/LastPixel are inclusive positions inside the current eight-pixel slice. OutputPixel is the first screen-relative graphics position of the span before border masking. Output*, Pending*, and Staged* expose the complete graphics/matrix/color triplet at each sequencer stage. ReloadPixel is the XSCROLL comparator position when the current span can reload pending data, or - when it cannot. Drawing sprite: sprite number and latched X,Y position.

Timing: c-access runs in phi2 of cycles 15..54 and prepares the matrix/color entry for the following g-access. g-access runs in phi1 of cycles 16..55, stages its graphics/matrix/color triplet, is logged before VC/VLMI/GAccessIndex advance, and therefore precedes c-access when both occur in one recovered CPU cycle. After the complete current output slice has been consumed, the staged triplet replaces the pending input latch for the following cycle; consequently g-accesses 16..55 feed output slices 17..56. Drawing pixels is appended before its span advances the persistent shift register. A steady slice emits one span 0..7; a visual CPU register write emits span 0..3 with the state captured before the write and span 4..7 with the state captured after it. Drawing sprite is emitted when the horizontal comparator latches: pixels 0..3 are compared before the CPU write and pixels 4..7 after it.

Events: Cycle 15 contains only the first c-access; cycles 16..54 log g-access and then c-access; cycle 55 contains only the final g-access. Invalid=1 identifies a DMA-delay/FLI c-access that supplied fallback bus data. XSCROLL selects ReloadPixel, which transfers the complete pending triplet from the preceding cycle to the active output stage; the active byte can therefore continue into the following VIC-II cycle. A pending byte whose comparator is missed is replaced when the current cycle's staged g-access commits after drawing. The shift stream advances even where graphics are outside pixels 0..319 or later hidden by the border. Once a sprite X position is latched, a later write cannot reposition its running sequencer.

## VICII — CPU transaction prediction

Producer: src/COMMODORE/VICII/VICII.cpp :: debugCPUStopPrediction

Fields: Transaction and bus structure; raster origin; current and following BA/AEC windows with badline/sprite provenance; DMA masks; stop prediction and write-effect positions.

Timing: Emitted when instruction or interrupt context is received, before its cycles are recovered.

Events: CPU stop recalculation reports Reason=BadLineChange or SpriteDMAChange and serialises the windows/prediction again.

## VICII — CPU stop applied / VIC-II register write

Producer: src/COMMODORE/VICII/VICII.cpp

Fields: Applied cycle, raster position, duration, and first stop cycle; or address/register/value, write index, and pending writes.

Timing: The register-write event is emitted during the CPU phase of the reported absolute cycle, after the VIC-II phi1 access and any stolen phi2 c-access, and before final pixel/IRQ processing.

Events: Its position after all memory-access events in the same Info Cycle makes the effective phase order explicit.

## CIA1/CIA2 — Info Cycle

Producer: src/COMMODORE/CIA/CIA.cpp :: debugCIACycle

Fields: Each port: output latch, DDR, and effective hexadecimal value. Each timer: ON/OFF, count mode, initial latch, and current counter.

Timing: cpu.clockCycles()-i for each recovered cycle.

Events: CIA1 adds Additional Info with the keyboard matrix, last pressed/released key, and joystick registers.

## SID — Info Cycle

Producer: src/COMMODORE/SID/SID.cpp :: debugSIDCycle

Fields: Voice N: ACTIVE, ADSR, and three Wave n=TYPEANDFREQUENCY descriptions.

Timing: cpu.clockCycles()-i. Describes the current sound wrapper, both simple and reSID.

Events: Adds no subordinate events.

## PLA / C64 Memory

Producer: src/C64/PLA.cpp and src/C64/Memory.cpp

Fields: PLA: five on/off logic lines. C64 Memory: per subset (active, active for reading).

Timing: PLA uses the current CPU cycle; Change Configuration uses cycle -.

Events: The state reflects the implemented configuration after changes to the 6510 port or cartridge lines.

6. Interpretation method

# 6.1 Recommended procedure

- Identify the block delimited by + and note the absolute CPU cycle.

- Read Transaction about to execute and the PC line to establish the instruction, mode, and nominal bus structure.

- Count each chip's ordinary cycles, excluding Additional Info and projection records.

- Correlate subordinate events with the snapshot they follow and check their effect in the next cycle.

- For interrupts, build the sequence request → wait/rejection → launch → CPUINT → effective entry.

- Record observed facts, inferences, and timing limitations separately.

# 6.2 Annotated example

CPU     171065245   Info Cycle
                    Last 4 cycles
CPU     171065245   Transaction about to execute
                    Transaction         :Type=Instruction,Address=$1222,Code=208,Cycles=2
VICII   171065245   CPU transaction prediction
                    Raster origin       :Row=297,Cycle=17
VICII   171065245   Info Cycle
                    CPU bus ownership   :BALow=0,AECLow=0,...
                    Raster position     :Column=28(128),Row=297(297),17
                    Reading Graphics [Address=$30ca,Data=3c,Screen=19,Color=1,GAccessIndex=1,VLMI=1,VC=41,RC=2,Idle=0,Mode=0,CharacterBase=$3000,BitmapBase=$2000]
                    Drawing pixels [Raster=32,FirstPixel=0,LastPixel=7,XScroll=3,OutputPixel=8,ShiftActive=1,ShiftPixel=5,OutputData=7e,OutputScreen=18,OutputColor=1,PendingReload=1,PendingData=3c,PendingScreen=19,PendingColor=1,StagedAccess=1,StagedData=55,StagedScreen=1a,StagedColor=1,ReloadInSpan=1,ReloadPixel=3]
+

Reduced extract from emulators/N.LOG. Ellipses replace fields to keep the example focused; the preceding catalog defines the current output.

7. Limitations and maintenance

# 7.1 Specific limitations

- A block may contain register values already modified by a complete instruction even when the recovered VIC-II cycle is earlier within that instruction.

- Windows are serialised as FirstBA/FirstAEC/LastCycle/BadLineSource/SpriteSourceMask, with multiple windows separated by |.

- For bad lines, BadlineCAccessStartCycle is the latch cycle, BadlineFirstCAccessCycle is the first attempted access, BadlineCAccessStartedFromIdle preserves the sequencer state at recognition, and BadlineBARequestCycle is the first effective BA cycle.

- CIA1 Additional Info does not represent an additional cycle.

- Sprite and pixel events can make the file grow substantially when VIC-II is selected.

# 7.2 Maintenance contract

- The current code is the semantic authority, not an old LOG file or help text.

- Every change to a DEBUG producer requires review of debug-common.yaml or the relevant machine source and regeneration of the DOCX files.

- check_debug_docs.py compares the structured sources, watched C++ files, and generated artefacts with generated-state.json.

- Items marked TODO exist in the emulator assembly but currently write no fields; they are documented to avoid promising output that does not exist.

Appendices

# Appendix A. Provenance and revision

Structured sources: .codex/documentation/debug-files/sources/debug-common.yaml and .codex/documentation/debug-files/sources/c64.yaml.

# A.1 Watched source files

| Path | Scope |
| --- | --- |
| include/CORE/DebugFile.hpp | Common |
| src/CORE/DebugFile.cpp | Common |
| src/CORE/Computer.cpp | Common |
| src/CORE/CPU.cpp | Common |
| src/CORE/CPUInterrupt.cpp | Common |
| src/CORE/Memory.cpp | Common |
| src/CORE/StdCommands.cpp | Common |
| src/CONSOLE/LocalConsole.cpp | Common |
| src/C64/C64.cpp | c64 |
| src/C64/PLA.cpp | c64 |
| src/C64/CIA1.cpp | c64 |
| src/C64/Memory.cpp | c64 |
| src/C64/Cartridge.cpp | c64 |
| src/COMMODORE/VICII/VICII.cpp | c64 |
| src/COMMODORE/CIA/CIA.cpp | c64 |
| src/COMMODORE/SID/SID.cpp | c64 |
| src/COMMODORE/1530Datasette.cpp | c64 |
| src/COMMODORE/SerialIOPeripherals.cpp | c64 |

# A.2 Revision history

| Version | Year | Description |
| --- | --- | --- |
| 1.0 | 2026 | Initial inventory generated and checked against the current C++ producers. |

# Illustrations

Actualice los campos para generar la lista de ilustraciones.

# Equations

Actualice los campos para generar la lista de ecuaciones.