ZX80 and ZX81 DEBUG File

EMULATORS deep-debug file format

Ignacio Cea Forniés

2026

# Contents

Contents	1

Foreword	3

1. Purpose, scope, and models	4

1.1 Scope of this guide	4

2. Activation and producer selection	5

2.1 Console command	5

2.2 Conditions and special paths	5

3. Common file structure	6

3.1 Physical hierarchy	6

3.2 Timing rules	6

4. Common framework records	7

CPU — Info Cycle	7

CPU — Transaction about to execute	7

CPU — Executed instruction	7

CPU — Stop	7

CPU — Interrupts	8

Memory — Applied Set	8

5. Machine-specific components	9

5.1 Selectable inventory	9

5.2 Record catalog	9

ULA — Info Cycle	9

ULA — Port Read	9

ULA — Character Capture	10

ULA — Character Load	10

ULA — Line Advance	10

ZX81DN — Datasette trap executed	10

ZX81PRT — Printer trap executed	11

ULA — Line Sync	11

ULA — INT Response / Horizontal Reset / HSync Start / HSync End	11

CPU — NMI Synchronization	11

6. Interpretation method	13

6.1 Recommended procedure	13

6.2 Annotated example	13

7. Limitations and maintenance	14

7.1 Specific limitations	14

7.2 Maintenance contract	14

Appendices	15

Appendix A. Provenance and revision	15

A.1 Watched source files	16

A.2 Revision history	16

Illustrations	16

Equations	17

Foreword

This guide defines the DEBUG format for ZX80 and ZX81. Its purpose is to make a trace readable and auditable directly against the code that produces it. Audience: Framework and emulator developers, timing-test authors, and users diagnosing execution traces.

1. Purpose, scope, and models

Provide a verifiable description of the format, provenance, timing, and interpretation of EMULATORS deep-debug files.

# 1.1 Scope of this guide

Covered models: ZX80 and the PAL/NTSC ZX81 variants supported by the ZX81 executable.

Main processor: Zilog Z80.

The guide describes the behavior implemented in the current repository revision. When a component is present but its producer contains a TODO, the guide explicitly states that it currently produces no output.

2. Activation and producer selection

# 2.1 Console command

DEBUGON FILENAME YES|NO [YES|NO] [D:DIR1-DIR2] [ALLCHIP|C:id ...] [ALLIO|I:id ...] [ALLMEM|M:id ...]
DEBUGOFF
IDEBUGON FILENAME [YES|NO]
IDEBUGOFF

| Parameter | Meaning |
| --- | --- |
| FILENAME | Output file path. |
| First YES¦NO | YES connects the CPU to DEBUG; NO omits CPU records. |
| Second YES¦NO | YES appends to the file; NO creates or truncates it. The default is NO. |
| D:start-end | Restricts CPU records to the stated PC range. It does not by itself filter selected chips. |
| ALLCHIP / C:id | Selects every chip or specific identifiers. |
| ALLIO / I:id | Selects every device or specific identifiers. |
| ALLMEM / M:id | Selects every memory unit or specific identifiers. |

# 2.2 Conditions and special paths

- DEBUGON and IDEBUGON are mutually exclusive.

- Deactivation is deferred when requested while an instruction is executing, so that a block is not cut in half.

- The canonical internal command is CACTIVATEDEEPDEBUG; DEBUGON is its console alias. DEBUGOFF is an alias for CDESACTIVATEDEEPDEBUG.

- An additional path exists: when peripheral data is loaded with a third parameter, the console can activate an appended CPU trace without explicitly selecting chips, devices, or memory.

3. Common file structure

# 3.1 Physical hierarchy

| Element | Representation |
| --- | --- |
| Complete header | COMPONENT + absolute cycle + record class. Component and cycle are fitted into 8- and 12-character columns. |
| Subordinate data | Five tabs of indentation. An attribute uses a name fitted to 20 characters, a colon, and its value. |
| Simple text | Written without columns or indentation; it is used, for example, in the IDEBUGON preamble. |
| Direct write | Some older producers build tab-separated lines with operator<< and do not guarantee exactly the same column widths. |
| Block separator | A line containing only + closes one complete computer iteration. It is emitted after CPU, chip, and I/O processing while DEBUG is active. |

# 3.2 Timing rules

- The header cycle is the emulator's absolute counter unless the machine-specific section documents an anomaly or different unit.

- In complete-instruction mode, the CPU finishes the instruction before chips recover their cycles. Membership of a block does not prove the exact physical cycle of a mapped write.

- Chip snapshots are normally written before the recovered cycle is processed. A subordinate event occurs during that cycle, and its resulting state is usually visible in the next snapshot.

- Additional Info records do not count as ordinary cycles when checking instruction duration.

- Duration comparisons must be restarted around stop blocks or interrupt entry.

- An isolated PC jump does not prove an interrupt; explicit evidence or reconstruction from the vector and stack is required.

4. Common framework records

# CPU — Info Cycle

Producer: src/CORE/CPU.cpp :: CPU::debugLastExecutionData / debugInstructionWaiting

Condition: At the start of a CPU iteration, or while waiting for an instruction in cycle-by-cycle mode.

Fields: Absolute cycle; Last N cycles, or Instruction in N cycles.

Interpretation: Last N cycles belongs to the previous instruction. The following PC line describes the instruction that has just executed in the current block. Duration is compared between consecutive blocks.

Status: Implemented

# CPU — Transaction about to execute

Producer: src/CORE/CPU.cpp :: CPU::debugInstructionAboutToExecute / debugInterruptAboutToExecute

Condition: Before consuming an instruction transaction or launching an interrupt.

Fields: Transaction(Type, Address/Id, Code, Cycles); Bus cycle data(Defined, Structure, Reads, Writes, TrailingWrites, LastCycleType); Execution(Mode, State, LastState).

Interpretation: Exposes the selected nominal bus structure. LastCycleType uses CPUCycle flags: read=2 and write=4; interpret other values according to CPUTransaction.hpp.

Status: Implemented

# CPU — Executed instruction

Producer: src/CORE/CPU.cpp :: CPU::debugInstructionExecuted

Condition: After a complete instruction has executed successfully.

Fields: Previous PC and stack position; disassembly; IN/OUT address and data when present; State/Last State; status register; CPU registers.

Interpretation: The parenthesised annotation is the effective address and the data observed or transferred by the instruction, not a separate new record.

Status: Implemented

# CPU — Stop

Producer: src/CORE/CPU.cpp :: debugStopRequest / debugStopSituation / debugAlreadyStopped

Condition: Stop request, entry into the stopped state, and later cycles still covered by the stop.

Fields: Serialised StopStatusData; State; LastState; number of pending cycles.

Interpretation: Stop request is the request; Stopped describes the applied state; Already stopped confirms that the validity counter remains active.

Status: Implemented

# CPU — Interrupts

Producer: src/CORE/CPU.cpp and src/CORE/CPUInterrupt.cpp

Condition: Request, wait, rejection, launch, failure, or start of interrupt code.

Fields: Serialised request; Id; Priority; LaunchCycles; bus structure; Clock and PC.

Interpretation: Repeated requests while a line remains active do not prove repeated edges. Correlate them with wait, not allowed, launched, and CPUINT records.

Status: Implemented

# Memory — Applied Set

Producer: src/CORE/Memory.cpp :: SetMemoryCommand::execute

Condition: Application of a memory write stored as a command.

Fields: Subset name; cycle shown as -; hexadecimal Address; hexadecimal Value.

Interpretation: The dash means that the producer supplies no absolute cycle. The write is documented when the command is applied to the subset.

Status: Implemented

5. Machine-specific components

# 5.1 Selectable inventory

| Id | Component | DEBUG output |
| --- | --- | --- |
| 210 | ULA | Raster, sync, line counter, video shift register, and port read results |
| 100/101 | Datasette | Tape port and load simulation |
| 102 | Thermal printer | Simulated printing trap |
| Memory | ZX80/ZX81 map | Applied Set for selected subsets |

# 5.2 Record catalog

## ULA — Info Cycle

Producer: src/ZX81/ULA.cpp :: debugULACycle

Fields: Raster position: zero-based column and line. Internal status: NMI=NMI_ON|NMI_OFF, ZONE=WHITE|BLACK, LNCTRL (0-7), LineSyncActive (0/1). Shift register: Data (decimal byte), PendingBits (0-8), Inverse (0/1). Timing: PixelPhase (0/1), State=BeforeShift.

Timing: Header clock is _lastCPUCycles + (i >> 1), in CPU T-states. PixelPhase distinguishes the two pixels per T-state. The snapshot follows due horizontal-generator events and pattern loads, before pixel shifting and presentation advance. It contains no per-pixel CPU snapshot or CPUClock field.

Events: ZONE names the syncOutputWhite presentation gate, not a physical sync voltage or noise source. LineSyncActive describes the ZX80 coarse line model; ZX81 hardware timing uses separate generator records.

## ULA — Port Read

Producer: src/ZX81/ULA.cpp :: debugPortRead, called by src/ZX81/PortManager.cpp :: getValue

Fields: Raster position: Column and Row (zero-based). Port: Access=READ|PEEK, Address (full 16-bit address), ID (port selector), Data (returned byte), D6 and D7. Internal status: NTSC, EAR, MIC, VSYNC, LNCTRL and LNBlocked. Timing: Reference=LastULASimulatedCPUClock, State=AfterRead. All numbers are decimal; flags are 0/1.

Timing: Emitted at the end of every getValue call, including odd ports and PEEK, only when ULA deep debug is active. Header time is _lastCPUCycles: the last CPU cycle processed by the ULA, not the exact I/O sample time. State is observed after the read side effects.

Events: For even ports D6 preserves the regional selection and D7 carries EAR. With no keys pressed, PAL returns 127 or 255 for EAR=0 or 1; NTSC returns 63 or 191. Odd-port data comes from lastVRAMByteRead and has no such bit interpretation. PEEK identifies monitor inspection and must not be counted as an executed IN instruction.

## ULA — Character Capture

Producer: src/ZX81/ULA.cpp :: debugCharCapture

Fields: Character: Code, I. Refresh: R and Address (full 16-bit I:R). Timing: CaptureClock, ObservedClock, LoadClock, LoadPhase, DelayPixels, Reference=InstructionFetch. Numbers are decimal.

Timing: CaptureClock is the logical M1 start; ObservedClock is the CPU clock when capture is reported. They can differ and log emission order need not follow CaptureClock order. Refresh.Address preserves I:R before this M1 increments R, including I0 and R7. No increment is anticipated and CPU state is not modified. Earlier prefix increments are already present. The second M1 is attributed to instruction start + 4 T, excluding WAIT states. PerCycle can report it late; timestamping does not replay already simulated ULA cycles or fix raw-byte cycle prediction.

Events: Each queued capture retains its own refresh address despite subsequent CPU changes. Capture is independent of RAM configuration; the load later consults the active expansion capability.

## ULA — Character Load

Producer: src/ZX81/ULA.cpp :: debugCharLoad

Fields: Raster position: Column, Row. Character: Code, I, LNCTRL. Refresh: R, Address. Pattern: Source=RAMRefresh|CharacterPath, Address, Data, Accepted (NA before load, 0/1 afterwards). Shift register: Data, PendingBits, Inverse. Timing: CaptureClock, ScheduledClock, ScheduledPhase, ActualClock, ActualPhase, Late, Stage=BeforeLoad|AfterLoad. Numbers are decimal.

Timing: Header is the actual load clock. Refresh.R is the pre-increment value retained from the captured M1, not the live CPU register at load time. ScheduledPhase and ActualPhase distinguish the two pixels per T-state.

Events: Refresh.Address retains I:R including I0. Source=RAMRefresh selects this address when the active 16K expansion supports refresh reads, including its ULA-view mirror. Source=CharacterPath retains the previous character-formula address otherwise; it does not necessarily mean ROM. Pattern.Address is the effective address read. This fallback preserves compatibility and does not model an undriven refresh bus. Rejected loads are consumed, not rescheduled.

## ULA — Line Advance

Producer: src/ZX81/ULA.cpp :: debugLineAdvance, called by ULA::simulate

Fields: Raster position: HBefore,HAfter,VBefore,VAfter. Internal status: LNCTRL,PendingBits. Timing: PixelPhase,State=AfterHorizontalWrap.

Timing: Emitted after a presentation horizontal wrap and vertical advance. Does not increment LINECTRL or request NMI on ZX81.

Events: Presentation only. INT response alignment can suppress a coincident wrap. Compare separate hardware generator records.

## ZX81DN — Datasette trap executed

Producer: src/ZX81/Datasette.cpp :: debugSimulation

Fields: ZX81DN header, CPU cycle, and the literal Datasette trap executed.

Timing: Emitted when tape-routine simulation is applied.

Events: Contains no additional attributes.

## ZX81PRT — Printer trap executed

Producer: src/ZX81/StdPrinter.cpp :: debugSimulation

Fields: ZX81PRT header, CPU cycle, and the literal Printer trap executed.

Timing: Emitted when the printer routine is intercepted.

Events: The device manages printed content; this header confirms the trap.

## ULA — Line Sync

Producer: src/ZX81/ULA.cpp :: debugLineSync

Fields: Raster position: HBefore,HAfter,Row. Cause: ExternalSync,InternalSync,AlreadyActive,Applied,INTackClock. Internal status: LNBefore,LNAfter,LNBlocked,NMIGenerator,PendingBits. Timing: PixelPhase,TailPixels,State=AfterLineSync.

Timing: ZX80 only; retains the existing coarse line model.

Events: Not a ZX81 hardware sync record.

## ULA — INT Response / Horizontal Reset / HSync Start / HSync End

Producer: src/ZX81/ULA.cpp :: debugHorizontalTiming

Fields: Generator: Before,After (ULA clocks),HSync,ResetPending. Raster position: Column,Row. Internal status: LNBefore,LNAfter,LNBlocked,NMIGenerator,NMIRequested. Timing: PixelPhase,Cause,ResponseClock,ResetClock,State=AfterEvent. Flags 0/1; numeric fields decimal.

Timing: ZX81 only. Baseline period 414 ULA clocks; start at 32, end at 64. INT Response schedules reset two CPU T-states later at phase 0. Reset wins over a simultaneous sync edge. HSync Start updates LINECTRL before character load and may request NMI. HSync End may also be caused by INT acknowledgement. InitializationOrVSync has only the last simulated ULA clock reference, not an exact bus time; ResponseClock/ResetClock are not applicable to that cause.

Events: Causes: CPUResponse,INTAcknowledge,Counter,InitializationOrVSync. ResponseClock is the CPU response-start timestamp, not /IORQ. Generator wraps independently of raster; terminal wrap creates no sync edge. HALT/NMI response compensation is reported separately by CPU NMI Synchronization; ordinary-instruction WAIT remains unmodeled. Initial counter 4 corresponds to presentation H=0; this preserves the existing coarse VSYNC restart and text-coordinate mapping. 207 T is the compatibility baseline, not a resolution of the 207/208 T hardware question.

## CPU — NMI Synchronization

Producer: src/ZX81/CZ80.cpp :: debugNMISynchronization

Fields: Request: RequestClock,AcceptanceClock. Synchronization: HALTBefore,Latency,AdditionalCycles. Model: Reference=HALT4T,Applied,Reason. Timing: NominalCycles,TotalCycles,State=BeforeResponseExecution. All clocks/durations are CPU T-states; flags are 0/1.

Timing: Emitted by CPU deep debug at acceptance of NMI from the linked ULA with reason 1, before generic interrupt execution changes HALT or PC. The generic transaction record still reports the nominal 11 T. Full mode extends the elapsed response duration; PerCycle mode extends remaining response cycles once.

Events: HALTed ZX81 with latency 0..4 receives 17-Latency additional cycles, the coarse four-T phase compensation compared with picozx81. NotHalted,LatencyOutsideModel,ModelNotSupported record exclusions, not proof of zero physical WAIT. PhaseCompensation identifies application. No ordinary-instruction WAIT, exact pin-level WAIT, or CPU/chip batch interleaving is implemented. ZX80 receives no additional cycles.

6. Interpretation method

# 6.1 Recommended procedure

- Identify the block delimited by + and note the absolute CPU cycle.

- Read Transaction about to execute and the PC line to establish the instruction, mode, and nominal bus structure.

- Count each chip's ordinary cycles, excluding Additional Info and projection records.

- Correlate subordinate events with the snapshot they follow and check their effect in the next cycle.

- For interrupts, build the sequence request → wait/rejection → launch → CPUINT → effective entry.

- Record observed facts, inferences, and timing limitations separately.

# 6.2 Annotated example

ULA     12000       Port Read
                    Raster position     :Column=0,Row=0
                    Port                :Access=READ,Address=32766,ID=254,Data=127,D6=1,D7=0
                    Internal status     :NTSC=0,EAR=0,MIC=0,VSYNC=1,LNCTRL=7,LNBlocked=1
                    Timing              :Reference=LastULASimulatedCPUClock,State=AfterRead
ULA     12000       Info Cycle
                    Raster position     :0,0
                    Internal status     :NMI=NMI_OFF,ZONE=BLACK,LNCTRL=7,LineSyncActive=0
                    Shift register      :Data=0,PendingBits=0,Inverse=0
                    Timing              :PixelPhase=0,State=BeforeShift
+
ULA     12207       Line Advance
                    Raster position     :HBefore=413,HAfter=0,VBefore=100,VAfter=101
                    Internal status     :LNCTRL=3,PendingBits=0
                    Timing              :PixelPhase=1,State=AfterHorizontalWrap
+

Schematic independent examples based on current producers; values are illustrative, not a captured execution trace.

7. Limitations and maintenance

# 7.1 Specific limitations

- ZX80 and ZX81 share one document because the executable and ULA class parameterise the type.

- The ULA derives its cycle from _lastCPUCycles and i>>1; it must not be replaced with cpu.clockCycles()-i.

- A tape or printer trap describes firmware simulation, not an electrical signal.

- CPU IN/OUT effects are released at transaction boundaries. The saved access clock does not schedule an exact intra-instruction ULA event. Port Read timestamps cannot measure exact I/O or VSYNC pulse widths: they use the last simulated ULA clock. The record shows post-read state and cannot reconstruct the raster before an IN reset.

- A zero Shift register Data byte does not prove that all pixels have shifted out; consult PendingBits. Character Load reports rejected loads; INT Response and Horizontal Reset identify distinct timing stages.

- PEEK preserves hardware state, MIC change notifications and screen event markers. It can emit a Port Read debug record, which is not an executed IN. Debug methods use non-consuming state getters.

- Line Advance reports presentation only. Use HSync Start and Horizontal Reset for ZX81 hardware-model timing; Line Sync is ZX80 only.

- PerCycle execution can batch several CPU boundaries before chips run. INT sampling/acceptance and the single acknowledgement latch therefore retain the documented coarse-scheduling limits.

- HALT/NMI compensation is a response-duration approximation; ordinary-instruction WAIT and exact pin-level WAIT remain unmodeled. Prefix captures use nominal M1 timestamps; PerCycle still selects later opcodes at execution completion and predicts duration from unmodified bytes.

- Instruction execution records can precede buffered input completion. Destination registers, input-dependent flags and block-input memory writes are completed during CPU buffer release; no dedicated port-release debug record is emitted yet.

# 7.2 Maintenance contract

- The current code is the semantic authority, not an old LOG file or help text.

- Every change to a DEBUG producer requires review of debug-common.yaml or the relevant machine source and regeneration of the DOCX files.

- check_debug_docs.py compares the structured sources, watched C++ files, and generated artefacts with generated-state.json.

- Items marked TODO exist in the emulator assembly but currently write no fields; they are documented to avoid promising output that does not exist.

Appendices

# Appendix A. Provenance and revision

Structured sources: .codex/documentation/debug-files/sources/debug-common.yaml and .codex/documentation/debug-files/sources/zx80-zx81.yaml.

# A.1 Watched source files

| Path | Scope |
| --- | --- |
| include/CORE/DebugFile.hpp | Common |
| src/CORE/DebugFile.cpp | Common |
| src/CORE/Computer.cpp | Common |
| src/CORE/CPU.cpp | Common |
| src/CORE/CPUInterrupt.cpp | Common |
| src/CORE/Memory.cpp | Common |
| src/CORE/StdCommands.cpp | Common |
| src/CONSOLE/LocalConsole.cpp | Common |
| src/ZX81/ZX81.cpp | zx80-zx81 |
| src/ZX81/ULA.cpp | zx80-zx81 |
| src/ZX81/Datasette.cpp | zx80-zx81 |
| src/ZX81/StdPrinter.cpp | zx80-zx81 |
| src/ZX81/PortManager.cpp | zx80-zx81 |
| include/ZX81/ULA.hpp | zx80-zx81 |
| include/ZX81/ULARegisters.hpp | zx80-zx81 |
| src/ZX81/CZ80.cpp | zx80-zx81 |
| include/ZX81/CZ80.hpp | zx80-zx81 |

# A.2 Revision history

| Version | Year | Description |
| --- | --- | --- |
| 1.0 | 2026 | Initial inventory generated and checked against the current C++ producers. |

# Illustrations

No se encuentran elementos de tabla de ilustraciones.

# Equations

No se encuentran elementos de tabla de ilustraciones.