Sinclair ZX80/ZX81 Emulator User Guide

ZX80 and ZX81 ROM variants: startup options and command console reference

Ignacio Cea Forniés

2026

# Contents

Contents	1

/o	6

/p	6

/h	6

/f	7

/k	7

/c	7

/l	8

/a	8

/d	8

/s	9

/i	9

/r	9

/g	10

/n	10

/w	10

/b	11

/m	11

/h	12

/f	12

/k	12

/c	13

/l	13

/a	13

/d	14

/s	14

/i	14

/r	15

/p	15

/g	15

/n	16

/w	16

/b	16

/m	17

3.1 Standard Framework Commands	18

HELP	18

AUTHOR	18

STATUS	18

PC	19

REGISTERS	19

CHANGEREG	19

STACK	20

CPUSTATUS	20

CPUSSTATUS	20

CPUSTATE	21

CPUINFO	21

MEMORY	21

MEMORYDUMP	22

MEMORYSTR	22

SETMEMORY	22

MEMORYANALYSIS	23

STOP	23

RUN	24

SETPC	24

NEXT	24

NEXTSTACK	25

SHOWNEXT	25

INST	25

BREAKS	26

SETBREAK	26

REMOVEBREAK	26

REMOVEBREAKS	27

SPEED	27

LOADBIN	27

SAVEBIN	28

DEBUGON	28

DEBUGOFF	28

IDEBUGON	29

IDEBUGOFF	29

RESTART	29

DEVICES	30

PERIPHERALS	30

PERCOMMAND	30

ASSIGNJOY	31

CLOCKFACTOR	31

SOUNDON	31

SOUNDOFF	32

INTERRUPTS	32

INTSET	32

CHIPS	33

CRTON	33

CRTOFF	33

DATASETTE	34

GRIDON	34

GRIDOFF	34

PICTURE	35

SETHOOK	35

REMOVEHOOK	35

HOOKS	36

HOOKSHELP	36

CMOVEPARAMS	36

3.2 Inherited Sinclair Commands	37

SYSVARS	37

SYSVAR	37

3.3 ZX80/ZX81-Specific Commands	38

ULA	38

ULAEVENTS	38

DFDUMP	38

CHARSDRAW	39

3.4 Local-Console-Only Commands	39

LOADPRG	39

LOADBINARY	39

LOADBLOCKS	40

DECOMPILE	40

POSSIBLEPERS	40

CONNECTPER	41

DISCONNECTPERS	41

LOADPERDATA	42

EMPTYPERDATA	42

SAVEPERDATA	42

CLEARPERDATA	43

Appendix A. Command Availability Summary	44

Appendix B. Source-of-Truth Maintenance Rule	44

Illustrations	45

Equations	46

Foreword

This guide describes the startup and command interfaces implemented by the two Sinclair ZX80/ZX81 executables in EMULATORS: ZX81Emulator (without a local command console) and ZX81EmulatorC (with a local command console). It covers the ZX80 and the three ZX81 ROM selectors and is based on both executable entry points, the ZX81::ZX81Emulator inheritance chain, LocalConsole, and the complete ZX81 -> SINCLAIR -> StandardCommandBuilder responsibility chain.

Command and parameter names are shown exactly as accepted by the current code. Startup options use /XVALUE with no space between the one-letter option and its value. Command names are matched as written by the builders; use uppercase names for reliable operation. Positional command parameters are separated by spaces; examples use paths without embedded spaces.

The non-console executable does not expose an interactive prompt. When started with /o it creates a remote communication system using the same ZX80/ZX81, Sinclair and standard command builders. Commands marked Local console only are intercepted by LocalConsole and therefore cannot be sent to the non-console executable.

1. Startup Parameters: Version Without a Local Console

Executable: ZX81Emulator.exe. It accepts the framework and ZX80/ZX81-specific options below, plus /o and the communication-port option. Option letters are case-sensitive because the command-line parser stores the character exactly as supplied.

# /o

Meaning: Controls creation of the remote command communication system.

General form:

/o[YES|NO]

- YES or an empty value: open remote communications.

- NO (and any other non-YES non-empty value): do not open it.

- Omitted: remote communications disabled.

Examples:

ZX81Emulator.exe /o

ZX81Emulator.exe /oYES /p60000

Consequence: When enabled, remote clients can execute ZX80/ZX81, Sinclair and standard builder commands. LocalConsole-only commands remain unavailable.

# /p

Meaning: Selects the TCP listening port used by the non-console executable's remote command channel.

General form:

/pPORTNUMBER

- 0 through 65535: requested listening port; values above 1000 are recommended.

- Omitted: port 60000 is used when /o enables the channel.

- Collision: the inherited emulator also interprets /p as a comma-separated peripheral-ID list. A supplied port number is therefore also attempted as a peripheral ID during initialization.

Examples:

ZX81Emulator.exe /o

ZX81Emulator.exe /o /p61000

Consequence: The remote server listens on the selected port. When /p is present, the same value is also passed to peripheral startup and normally produces a log warning because a port number is not a valid peripheral ID.

# /h

Meaning: Prints the executable help and prevents the emulation from starting.

General form:

/h

- No value.

Examples:

ZX81Emulator.exe /h

Consequence: Initialization returns failure after printing the help; the current main program therefore exits with code 1.

# /f

Meaning: Loads a binary file whose header contains the destination address.

General form:

/fFILE

- FILE: path to the header-addressed binary file; attach it directly to /f.

Examples:

ZX81Emulator.exe /fprogram.bin

ZX81Emulator.exe /fgames\demo.bin /a$080d

Consequence: The file is loaded before execution. An unreadable or invalid file aborts initialization.

# /k

Meaning: Loads a block-structured memory file.

General form:

/kFILE

- FILE: path to a block file understood by the framework.

Examples:

ZX81Emulator.exe /ksnapshot.blocks

Consequence: Every encoded block is placed at its stored address. A load error aborts initialization.

# /c

Meaning: Parses, assembles and loads an assembler source file.

General form:

/cFILE

- FILE: assembler source path.

Examples:

ZX81Emulator.exe /cstartup.asm /s

Consequence: The assembled bytes are installed before the main loop. Syntax or load errors are logged and initialization stops.

# /l

Meaning: Selects the framework diagnostic level.

General form:

/lLEVEL

- 0: no diagnostics.

- 1: errors (default).

- 2: dump at exit.

- 3: errors and warnings.

- 4: all regular diagnostics.

- 5: internal trace diagnostics.

Examples:

ZX81Emulator.exe /l0

ZX81Emulator.exe /l4

Consequence: Higher values produce progressively more diagnostic output and may reduce performance.

# /a

Meaning: Overrides the address at which execution begins.

General form:

/aADDRESS

- ADDRESS: ZX80/ZX81 address in the framework number syntax, for example decimal or hexadecimal with $.

Examples:

ZX81Emulator.exe /a$080d

ZX81Emulator.exe /fprogram.bin /a2061

Consequence: A non-zero, valid address replaces the normal reset/program entry address.

# /d

Meaning: Creates startup stop points at one or more addresses.

General form:

/dADDRESS1,ADDRESS2,...

- ADDRESSn: comma-separated address. Each entry accepts the framework number syntax.

Examples:

ZX81Emulator.exe /d$080d

ZX81Emulator.exe /d$080d,$ffd2 /s

Consequence: Execution stops when the CPU reaches any listed address, allowing later inspection or continuation.

# /s

Meaning: Controls whether the emulated computer starts stopped.

General form:

/s[YES|NO]

- YES or an empty value: start stopped.

- NO (and any non-YES non-empty value): start running.

- Omitted: start running.

Examples:

ZX81Emulator.exe /s

ZX81Emulator.exe /sNO

Consequence: When enabled, the machine is initialized but the CPU does not run until a RUN/NEXT command or the equivalent UI action is issued.

# /i

Meaning: Selects the language identifier exposed to the emulated machine.

General form:

/iLANG

- ENG: English; this is the only language advertised and supported by the ZX80/ZX81 emulator.

Examples:

ZX81Emulator.exe /iENG

Consequence: The machine starts with the English language identifier. Other values are passed through but have no documented ZX80/ZX81 ROM variant in this implementation.

# /r

Meaning: Requests the initial sound-output state.

General form:

/r[YES|NO]

- YES or an empty value: requests sound enabled.

- NO (and any other non-YES non-empty value): requests sound disabled.

- Omitted: requests sound enabled.

Examples:

ZX81Emulator.exe /r

ZX81Emulator.exe /rNO

Consequence: The option is parsed, but the implemented ZX80/ZX81 computer has no sound device, so initialization makes no audio-state change.

# /g

Meaning: Controls the CRT post-processing effect at startup.

General form:

/g[YES|NO]

- YES or an empty value: CRT effect enabled.

- NO (and any non-YES non-empty value): disabled.

- Omitted: disabled.

Examples:

ZX81Emulator.exe /g

ZX81Emulator.exe /gNO

Consequence: The screen starts with or without CRT emulation. CRTON and CRTOFF can change it later.

# /n

Meaning: Requests NTSC visualization parameters.

General form:

/n

- No value: the presence of the option requests NTSC.

Examples:

ZX81Emulator.exe /mZX813 /n

Consequence: The current factory always constructs PAL ZX80 and ZX81 machines. The option is detected and a limitation is written to the log, but PAL timing and screen geometry remain active.

# /w

Meaning: Selects the startup RAM configuration.

General form:

/wCONF

- ZX80: only configuration 0 (unexpanded) is implemented; another value is ignored and logged.

- ZX81 ROM variants, 0 or omitted: unexpanded configuration (default).

- ZX81 ROM variants, 1: 16 KiB expansion.

- ZX81 ROM variants, any value other than 0 or 1: reset to unexpanded configuration and logged.

Examples:

ZX81Emulator.exe /mZX80 /w0

ZX81Emulator.exe /mZX813 /w1

Consequence: The selected memory map is installed before the ROM and machine initialize. The broader 3K/8K/24K list in an old header comment is not implemented by the current factory.

# /b

Meaning: Draws a diagnostic grid around the writable screen area and selects its color.

General form:

/b[COLOR]

- COLOR omitted or 0: black (default).

- 1 through 15: the requested grid color.

- Any value greater than 15: clamped to 15.

Examples:

ZX81Emulator.exe /b

ZX81Emulator.exe /b14

Consequence: The screen is initialized with the diagnostic grid enabled in the selected color.

# /m

Meaning: Selects the ZX80 or ZX81 ROM variant to emulate.

General form:

/mMACHINE

- ZX80: ZX80 model and ROM; this is the default when /m is omitted.

- ZX811: ZX81 with the old ROM image.

- ZX812: ZX81 with the rare/intermediate ROM image.

- ZX813: ZX81 with the newest ROM image.

- Any other value: logs the unsupported selector and falls back to ZX80.

Examples:

ZX81Emulator.exe /mZX80

ZX81Emulator.exe /mZX811

ZX81Emulator.exe /mZX813 /w1

Consequence: The selector chooses the ROM image and machine type created before initialization. It does not independently select a ULA silicon revision, video standard or RAM expansion.

2. Startup Parameters: Version With a Local Console

Executable: ZX81EmulatorC.exe. Its main program delegates the complete command line to ZX81::ZX81Emulator, so all inherited framework options and ZX80/ZX81-specific options are active even though the short main-level help banner only advertises /h. It has no /o listener option; commands are entered in its local console.

# /h

Meaning: Prints the executable help and prevents the emulation from starting.

General form:

/h

- No value.

Examples:

ZX81EmulatorC.exe /h

Consequence: Initialization returns failure after printing the help; the current main program therefore exits with code 1.

# /f

Meaning: Loads a binary file whose header contains the destination address.

General form:

/fFILE

- FILE: path to the header-addressed binary file; attach it directly to /f.

Examples:

ZX81EmulatorC.exe /fprogram.bin

ZX81EmulatorC.exe /fgames\demo.bin /a$080d

Consequence: The file is loaded before execution. An unreadable or invalid file aborts initialization.

# /k

Meaning: Loads a block-structured memory file.

General form:

/kFILE

- FILE: path to a block file understood by the framework.

Examples:

ZX81EmulatorC.exe /ksnapshot.blocks

Consequence: Every encoded block is placed at its stored address. A load error aborts initialization.

# /c

Meaning: Parses, assembles and loads an assembler source file.

General form:

/cFILE

- FILE: assembler source path.

Examples:

ZX81EmulatorC.exe /cstartup.asm /s

Consequence: The assembled bytes are installed before the main loop. Syntax or load errors are logged and initialization stops.

# /l

Meaning: Selects the framework diagnostic level.

General form:

/lLEVEL

- 0: no diagnostics.

- 1: errors (default).

- 2: dump at exit.

- 3: errors and warnings.

- 4: all regular diagnostics.

- 5: internal trace diagnostics.

Examples:

ZX81EmulatorC.exe /l0

ZX81EmulatorC.exe /l4

Consequence: Higher values produce progressively more diagnostic output and may reduce performance.

# /a

Meaning: Overrides the address at which execution begins.

General form:

/aADDRESS

- ADDRESS: ZX80/ZX81 address in the framework number syntax, for example decimal or hexadecimal with $.

Examples:

ZX81EmulatorC.exe /a$080d

ZX81EmulatorC.exe /fprogram.bin /a2061

Consequence: A non-zero, valid address replaces the normal reset/program entry address.

# /d

Meaning: Creates startup stop points at one or more addresses.

General form:

/dADDRESS1,ADDRESS2,...

- ADDRESSn: comma-separated address. Each entry accepts the framework number syntax.

Examples:

ZX81EmulatorC.exe /d$080d

ZX81EmulatorC.exe /d$080d,$ffd2 /s

Consequence: Execution stops when the CPU reaches any listed address, allowing later inspection or continuation.

# /s

Meaning: Controls whether the emulated computer starts stopped.

General form:

/s[YES|NO]

- YES or an empty value: start stopped.

- NO (and any non-YES non-empty value): start running.

- Omitted: start running.

Examples:

ZX81EmulatorC.exe /s

ZX81EmulatorC.exe /sNO

Consequence: When enabled, the machine is initialized but the CPU does not run until a RUN/NEXT command or the equivalent UI action is issued.

# /i

Meaning: Selects the language identifier exposed to the emulated machine.

General form:

/iLANG

- ENG: English; this is the only language advertised and supported by the ZX80/ZX81 emulator.

Examples:

ZX81EmulatorC.exe /iENG

Consequence: The machine starts with the English language identifier. Other values are passed through but have no documented ZX80/ZX81 ROM variant in this implementation.

# /r

Meaning: Requests the initial sound-output state.

General form:

/r[YES|NO]

- YES or an empty value: requests sound enabled.

- NO (and any other non-YES non-empty value): requests sound disabled.

- Omitted: requests sound enabled.

Examples:

ZX81EmulatorC.exe /r

ZX81EmulatorC.exe /rNO

Consequence: The option is parsed, but the implemented ZX80/ZX81 computer has no sound device, so initialization makes no audio-state change.

# /p

Meaning: Connects one or more peripherals during startup.

General form:

/pPER1[,PER2...]

- 10: typewriter keyboard input.

- 100: Sinclair cassette signal simulation.

- 101: ZX80/ZX81 cassette-data injection.

- 102: ZX Printer thermal-printer simulation.

- 201: cartridge implementation supported by the builder, although it is not listed by POSSIBLEPERS.

Examples:

ZX81EmulatorC.exe /p10

ZX81EmulatorC.exe /p100,102

Consequence: Each requested peripheral is built and connected to the first compatible device during emulator initialization. Unsupported IDs are logged and initialization continues.

# /g

Meaning: Controls the CRT post-processing effect at startup.

General form:

/g[YES|NO]

- YES or an empty value: CRT effect enabled.

- NO (and any non-YES non-empty value): disabled.

- Omitted: disabled.

Examples:

ZX81EmulatorC.exe /g

ZX81EmulatorC.exe /gNO

Consequence: The screen starts with or without CRT emulation. CRTON and CRTOFF can change it later.

# /n

Meaning: Requests NTSC visualization parameters.

General form:

/n

- No value: the presence of the option requests NTSC.

Examples:

ZX81EmulatorC.exe /mZX813 /n

Consequence: The current factory always constructs PAL ZX80 and ZX81 machines. The option is detected and a limitation is written to the log, but PAL timing and screen geometry remain active.

# /w

Meaning: Selects the startup RAM configuration.

General form:

/wCONF

- ZX80: only configuration 0 (unexpanded) is implemented; another value is ignored and logged.

- ZX81 ROM variants, 0 or omitted: unexpanded configuration (default).

- ZX81 ROM variants, 1: 16 KiB expansion.

- ZX81 ROM variants, any value other than 0 or 1: reset to unexpanded configuration and logged.

Examples:

ZX81EmulatorC.exe /mZX80 /w0

ZX81EmulatorC.exe /mZX813 /w1

Consequence: The selected memory map is installed before the ROM and machine initialize. The broader 3K/8K/24K list in an old header comment is not implemented by the current factory.

# /b

Meaning: Draws a diagnostic grid around the writable screen area and selects its color.

General form:

/b[COLOR]

- COLOR omitted or 0: black (default).

- 1 through 15: the requested grid color.

- Any value greater than 15: clamped to 15.

Examples:

ZX81EmulatorC.exe /b

ZX81EmulatorC.exe /b14

Consequence: The screen is initialized with the diagnostic grid enabled in the selected color.

# /m

Meaning: Selects the ZX80 or ZX81 ROM variant to emulate.

General form:

/mMACHINE

- ZX80: ZX80 model and ROM; this is the default when /m is omitted.

- ZX811: ZX81 with the old ROM image.

- ZX812: ZX81 with the rare/intermediate ROM image.

- ZX813: ZX81 with the newest ROM image.

- Any other value: logs the unsupported selector and falls back to ZX80.

Examples:

ZX81EmulatorC.exe /mZX80

ZX81EmulatorC.exe /mZX811

ZX81EmulatorC.exe /mZX813 /w1

Consequence: The selector chooses the ROM image and machine type created before initialization. It does not independently select a ULA silicon revision, video standard or RAM expansion.

3. Command Console Reference

The local console checks its eleven emulator-level commands first. All remaining text is delegated through ZX81::CommandBuilder, then SINCLAIR::CommandBuilder, and finally MCHEmul::StandardCommandBuilder. The remote /o channel in ZX81Emulator.exe uses only that three-builder chain. The availability line in each entry makes this boundary explicit.

# 3.1 Standard Framework Commands

## HELP

Availability: Local console and remote /o channel.

Accepted aliases: CHELP.

Meaning: Displays the ZX80/ZX81 console help assembled from the ZX80/ZX81, Sinclair, local-console and standard help files.

General syntax:

HELP

Parameters:

- No parameters.

Examples:

HELP

Result and consequences: Displays the ZX80/ZX81 console help assembled from the ZX80/ZX81, Sinclair, local-console and standard help files.

## AUTHOR

Availability: Local console and remote /o channel.

Accepted aliases: CAUTHOR.

Meaning: Returns framework author information.

General syntax:

AUTHOR

Parameters:

- No parameters.

Examples:

AUTHOR

Result and consequences: Returns framework author information.

## STATUS

Availability: Local console and remote /o channel.

Accepted aliases: CSTATUS.

Meaning: Shows the Z80 status register.

General syntax:

STATUS

Parameters:

- No parameters.

Examples:

STATUS

Result and consequences: Shows the Z80 status register.

## PC

Availability: Local console and remote /o channel.

Accepted aliases: CPC.

Meaning: Shows the current program counter.

General syntax:

PC

Parameters:

- No parameters.

Examples:

PC

Result and consequences: Shows the current program counter.

## REGISTERS

Availability: Local console and remote /o channel.

Accepted aliases: CREGISTERS.

Meaning: Shows all CPU registers.

General syntax:

REGISTERS

Parameters:

- No parameters.

Examples:

REGISTERS

Result and consequences: Shows all CPU registers.

## CHANGEREG

Availability: Local console and remote /o channel.

Accepted aliases: CCHANGEREG.

Meaning: Changes one CPU internal register.

General syntax:

CHANGEREG REGNAME VALUE

Parameters:

- REGNAME: registered Z80 register name.

- VALUE: new decimal, z-prefixed binary, o-prefixed octal or $-prefixed hexadecimal value.

Examples:

CHANGEREG A $41

Result and consequences: The register changes only if it exists and accepts the supplied byte width; otherwise an error is returned.

## STACK

Availability: Local console and remote /o channel.

Accepted aliases: CSTACK.

Meaning: Shows stack state.

General syntax:

STACK [ALL]

Parameters:

- ALL: optional; includes stack contents in addition to basic state.

Examples:

STACK

STACK ALL

Result and consequences: Shows stack state.

## CPUSTATUS

Availability: Local console and remote /o channel.

Accepted aliases: CCPUSTATUS.

Meaning: Shows registers, status, program counter and stack.

General syntax:

CPUSTATUS

Parameters:

- No parameters.

Examples:

CPUSTATUS

Result and consequences: Shows registers, status, program counter and stack.

## CPUSSTATUS

Availability: Local console and remote /o channel.

Accepted aliases: CCPUSSTATUS.

Meaning: Shows the compact CPU status without stack detail.

General syntax:

CPUSSTATUS

Parameters:

- No parameters.

Examples:

CPUSSTATUS

Result and consequences: Shows the compact CPU status without stack detail.

## CPUSTATE

Availability: Local console and remote /o channel.

Accepted aliases: CCPUSTATE.

Meaning: Shows the CPU execution state and state-specific data.

General syntax:

CPUSTATE

Parameters:

- No parameters.

Examples:

CPUSTATE

Result and consequences: Shows the CPU execution state and state-specific data.

## CPUINFO

Availability: Local console and remote /o channel.

Accepted aliases: CCPUINFO.

Meaning: Returns the complete CPU information structure.

General syntax:

CPUINFO

Parameters:

- No parameters.

Examples:

CPUINFO

Result and consequences: Returns the complete CPU information structure.

## MEMORY

Availability: Local console and remote /o channel.

Accepted aliases: CMEMORY.

Meaning: Reads the active CPU-visible memory range.

General syntax:

MEMORY INITIAL [FINAL]

Parameters:

- INITIAL: first address.

- FINAL: optional last address; with no final address only the initial location is reported.

Examples:

MEMORY $0400

MEMORY $0400 $0427

Result and consequences: Reads the active CPU-visible memory range.

## MEMORYDUMP

Availability: Local console and remote /o channel.

Accepted aliases: CMEMORYDUMP.

Meaning: Dumps every memory view between two addresses, including inactive RAM/ROM views.

General syntax:

MEMORYDUMP INITIAL FINAL

Parameters:

- INITIAL: first address.

- FINAL: last address.

Examples:

MEMORYDUMP $a000 $a03f

Result and consequences: Dumps every memory view between two addresses, including inactive RAM/ROM views.

## MEMORYSTR

Availability: Local console and remote /o channel.

Accepted aliases: CMEMORYSTR.

Meaning: Shows the configured memory structure.

General syntax:

MEMORYSTR [ALL]

Parameters:

- ALL: optional; includes inactive regions. Without it only active regions are shown.

Examples:

MEMORYSTR

MEMORYSTR ALL

Result and consequences: Shows the configured memory structure.

## SETMEMORY

Availability: Local console and remote /o channel.

Accepted aliases: CSETMEMORY.

Meaning: Writes a value to one address or repeatedly across a range.

General syntax:

SETMEMORY INITIAL [FINAL] VALUE

Parameters:

- INITIAL: first address.

- FINAL: optional last address.

- VALUE: byte sequence in framework numeric syntax; multi-byte values advance by their own length.

Examples:

SETMEMORY $0400 $41

SETMEMORY $0400 $0427 $20

Result and consequences: The active memory view is modified immediately; unsafe writes can corrupt state or crash the emulated program.

## MEMORYANALYSIS

Availability: Local console and remote /o channel.

Accepted aliases: CMEMORYANALYSIS.

Meaning: Analyzes the current memory configuration.

General syntax:

MEMORYANALYSIS

Parameters:

- No parameters.

Examples:

MEMORYANALYSIS

Result and consequences: Analyzes the current memory configuration.

## STOP

Availability: Local console and remote /o channel.

Accepted aliases: CSTOP.

Meaning: Stops CPU execution.

General syntax:

STOP

Parameters:

- No parameters.

Examples:

STOP

Result and consequences: Stops CPU execution.

## RUN

Availability: Local console and remote /o channel.

Accepted aliases: R, CRUN.

Meaning: Starts or resumes CPU execution; alias R.

General syntax:

RUN [ADDRESS]

Parameters:

- ADDRESS: optional new program counter before running.

Examples:

RUN

R $080d

Result and consequences: The CPU resumes immediately, optionally after changing PC.

## SETPC

Availability: Local console and remote /o channel.

Accepted aliases: CSETPC.

Meaning: Sets the program counter without starting execution.

General syntax:

SETPC ADDRESS

Parameters:

- ADDRESS: new CPU address.

Examples:

SETPC $080d

Result and consequences: Subsequent execution continues from the new address; an unsafe address can break the program.

## NEXT

Availability: Local console and remote /o channel.

Accepted aliases: N, CNEXT.

Meaning: Executes the next instruction; alias N.

General syntax:

NEXT

Parameters:

- No parameters.

Examples:

NEXT

N

Result and consequences: Executes the next instruction; alias N.

## NEXTSTACK

Availability: Local console and remote /o channel.

Accepted aliases: NS, CNEXTSTACK.

Meaning: Runs until the stack pointer returns to its initial value; alias NS.

General syntax:

NEXTSTACK

Parameters:

- No parameters.

Examples:

NS

Result and consequences: Useful for stepping over a routine, although behavior depends on the routine's stack discipline.

## SHOWNEXT

Availability: Local console and remote /o channel.

Accepted aliases: SN, CSHOWNEXT.

Meaning: Disassembles upcoming instruction(s); alias SN.

General syntax:

SHOWNEXT [NUMBER]

Parameters:

- NUMBER: optional count; one instruction by default.

Examples:

SHOWNEXT

SN 8

Result and consequences: Disassembles upcoming instruction(s); alias SN.

## INST

Availability: Local console and remote /o channel.

Accepted aliases: CINST.

Meaning: Shows the last instruction executed.

General syntax:

INST

Parameters:

- No parameters.

Examples:

INST

Result and consequences: Shows the last instruction executed.

## BREAKS

Availability: Local console and remote /o channel.

Accepted aliases: CBREAKS.

Meaning: Lists all stop-point addresses.

General syntax:

BREAKS

Parameters:

- No parameters.

Examples:

BREAKS

Result and consequences: Lists all stop-point addresses.

## SETBREAK

Availability: Local console and remote /o channel.

Accepted aliases: CSETBREAK.

Meaning: Adds one or more breakpoints.

General syntax:

SETBREAK ADDRESS1 [ADDRESS2 ...]

Parameters:

- ADDRESSn: one or more addresses in framework numeric syntax.

Examples:

SETBREAK $080d $ffd2

Result and consequences: Adds one or more breakpoints.

## REMOVEBREAK

Availability: Local console and remote /o channel.

Accepted aliases: CREMOVEBREAK.

Meaning: Removes selected breakpoints.

General syntax:

REMOVEBREAK ADDRESS1 [ADDRESS2 ...]

Parameters:

- ADDRESSn: one or more existing breakpoint addresses.

Examples:

REMOVEBREAK $ffd2

Result and consequences: Removes selected breakpoints.

## REMOVEBREAKS

Availability: Local console and remote /o channel.

Accepted aliases: CREMOVEBREAKS.

Meaning: Removes every breakpoint.

General syntax:

REMOVEBREAKS

Parameters:

- No parameters.

Examples:

REMOVEBREAKS

Result and consequences: Removes every breakpoint.

## SPEED

Availability: Local console and remote /o channel.

Accepted aliases: CSPEED.

Meaning: Shows measured CPU and screen-I/O execution speed.

General syntax:

SPEED

Parameters:

- No parameters.

Examples:

SPEED

Result and consequences: Shows measured CPU and screen-I/O execution speed.

## LOADBIN

Availability: Local console and remote /o channel.

Accepted aliases: CLOADBIN.

Meaning: Loads a raw binary file at an explicit address.

General syntax:

LOADBIN FILE ADDRESS

Parameters:

- FILE: raw binary path.

- ADDRESS: first destination address.

Examples:

LOADBIN charset.bin $2000

Result and consequences: Bytes are copied into memory without an embedded load-address header.

## SAVEBIN

Availability: Local console and remote /o channel.

Accepted aliases: CSAVEBIN.

Meaning: Saves a memory range as raw binary.

General syntax:

SAVEBIN FILE SIZE ADDRESS

Parameters:

- FILE: output path.

- SIZE: byte count.

- ADDRESS: first source address.

Examples:

SAVEBIN screen.bin 1000 $0400

Result and consequences: Saves a memory range as raw binary.

## DEBUGON

Availability: Local console and remote /o channel.

Accepted aliases: CACTIVATEDEEPDEBUG.

Meaning: Enables deep execution debugging.

General syntax:

DEBUGON FILE CPU [APPEND] [FILTERS...]

Parameters:

- FILE: output log.

- CPU: YES includes CPU execution; NO omits it.

- APPEND: optional YES appends, NO overwrites (default).

- D:START-END or ALLDIR: address filter.

- C:IDs or ALLCHIP: chip filter.

- I:IDs or ALLIO: device filter.

- M:IDs or ALLMEM: memory filter.

Examples:

DEBUGON zx81.log YES NO D:$0800-$0fff ALLCHIP ALLIO ALLMEM

Result and consequences: Tracing begins with the selected sources and can substantially slow emulation. It cannot coexist with interrupt-only debugging.

## DEBUGOFF

Availability: Local console and remote /o channel.

Accepted aliases: CDESACTIVATEDEEPDEBUG.

Meaning: Stops deep execution debugging.

General syntax:

DEBUGOFF

Parameters:

- No parameters.

Examples:

DEBUGOFF

Result and consequences: Stops deep execution debugging.

## IDEBUGON

Availability: Local console and remote /o channel.

Accepted aliases: CIDEBUGON.

Meaning: Enables interrupt-only debugging.

General syntax:

IDEBUGON FILE [APPEND]

Parameters:

- FILE: output log.

- APPEND: optional YES appends; NO overwrites (default).

Examples:

IDEBUGON interrupts.log NO

Result and consequences: Interrupt tracing begins and cannot coexist with deep debugging.

## IDEBUGOFF

Availability: Local console and remote /o channel.

Accepted aliases: CIDEBUGOFF.

Meaning: Stops interrupt-only debugging.

General syntax:

IDEBUGOFF

Parameters:

- No parameters.

Examples:

IDEBUGOFF

Result and consequences: Stops interrupt-only debugging.

## RESTART

Availability: Local console and remote /o channel.

Accepted aliases: CRESTART.

Meaning: Requests a restart and exits the current computer loop so it can be rebuilt.

General syntax:

RESTART STOPPED [LEVEL]

Parameters:

- STOPPED: YES restarts stopped; NO restarts running.

- LEVEL: optional 0 CPU-only/mild restart (default), 1 computer restart preserving memory, greater than 1 full restart.

Examples:

RESTART YES 0

RESTART NO 2

Result and consequences: Requests a restart and exits the current computer loop so it can be rebuilt.

## DEVICES

Availability: Local console and remote /o channel.

Accepted aliases: CDEVICES.

Meaning: Shows all or selected I/O devices.

General syntax:

DEVICES [ID1 ID2 ...]

Parameters:

- IDn: optional device identifiers; with none, all devices are returned.

Examples:

DEVICES

DEVICES 1 2

Result and consequences: Shows all or selected I/O devices.

## PERIPHERALS

Availability: Local console and remote /o channel.

Accepted aliases: CPERIPHERALS.

Meaning: Shows all or selected connected peripherals.

General syntax:

PERIPHERALS [ID1 ID2 ...]

Parameters:

- IDn: optional peripheral identifiers; with none, all are returned.

Examples:

PERIPHERALS

PERIPHERALS 100

Result and consequences: Shows all or selected connected peripherals.

## PERCOMMAND

Availability: Local console and remote /o channel.

Accepted aliases: CPERCMD.

Meaning: Sends a device-specific command to a connected peripheral.

General syntax:

PERCOMMAND PERID CMDID [ATTRS...]

Parameters:

- PERID: connected peripheral ID.

- CMDID: command ID implemented by that peripheral.

- ATTRS: optional positional attributes forwarded unchanged.

Examples:

PERCOMMAND 100 0

Result and consequences: The peripheral executes the request if both IDs and its parameter contract are valid; otherwise the answer reports an error.

## ASSIGNJOY

Availability: Local console and remote /o channel.

Accepted aliases: CASSIGNJ.

Meaning: Remaps a joystick identifier.

General syntax:

ASSIGNJOY ID NEWID

Parameters:

- ID: current joystick ID.

- NEWID: required target ID; using the same ID removes an existing assignment.

Examples:

ASSIGNJOY 0 1

Result and consequences: Remaps a joystick identifier.

## CLOCKFACTOR

Availability: Local console and remote /o channel.

Accepted aliases: CCLOCKFACTOR.

Meaning: Changes the CPU clock multiplier.

General syntax:

CLOCKFACTOR VALUE

Parameters:

- VALUE: numeric factor; 1 means normal speed.

Examples:

CLOCKFACTOR 2

Result and consequences: The emulation pacing target changes; values above one request faster execution.

## SOUNDON

Availability: Local console and remote /o channel.

Accepted aliases: CSOUNDON.

Meaning: Requests that sound output be enabled.

General syntax:

SOUNDON

Parameters:

- No parameters.

Examples:

SOUNDON

Result and consequences: The command is accepted, but the implemented ZX80/ZX81 computer has no sound device, so it produces no state change.

## SOUNDOFF

Availability: Local console and remote /o channel.

Accepted aliases: CSOUNDOFF.

Meaning: Requests that sound output be disabled.

General syntax:

SOUNDOFF

Parameters:

- No parameters.

Examples:

SOUNDOFF

Result and consequences: The command is accepted, but the implemented ZX80/ZX81 computer has no sound device, so it produces no state change.

## INTERRUPTS

Availability: Local console and remote /o channel.

Accepted aliases: CINTERRUPTS.

Meaning: Shows all interrupts known by the computer.

General syntax:

INTERRUPTS

Parameters:

- No parameters.

Examples:

INTERRUPTS

Result and consequences: Shows all interrupts known by the computer.

## INTSET

Availability: Local console and remote /o channel.

Accepted aliases: CINTSET.

Meaning: Enables or disables one interrupt or every interrupt.

General syntax:

INTSET ID|ALL ON|OFF

Parameters:

- ID or ALL: target interrupt.

- ON or OFF: requested state.

Examples:

INTSET ALL ON

Result and consequences: An unknown interrupt ID produces an error.

## CHIPS

Availability: Local console and remote /o channel.

Accepted aliases: CCHIPS.

Meaning: Lists the emulated chips and their identifying attributes.

General syntax:

CHIPS

Parameters:

- No parameters.

Examples:

CHIPS

Result and consequences: Lists the emulated chips and their identifying attributes.

## CRTON

Availability: Local console and remote /o channel.

Accepted aliases: CCRTON.

Meaning: Enables the CRT screen effect.

General syntax:

CRTON

Parameters:

- No parameters.

Examples:

CRTON

Result and consequences: Enables the CRT screen effect.

## CRTOFF

Availability: Local console and remote /o channel.

Accepted aliases: CCRTOFF.

Meaning: Disables the CRT screen effect.

General syntax:

CRTOFF

Parameters:

- No parameters.

Examples:

CRTOFF

Result and consequences: Disables the CRT screen effect.

## DATASETTE

Availability: Local console and remote /o channel.

Accepted aliases: CDATASETTE.

Meaning: Shows datasette state when a supported datasette is connected.

General syntax:

DATASETTE

Parameters:

- No parameters.

Examples:

DATASETTE

Result and consequences: Shows datasette state when a supported datasette is connected.

## GRIDON

Availability: Local console and remote /o channel.

Accepted aliases: CGRIDON.

Meaning: Enables the diagnostic grid overlay on the screen.

General syntax:

GRIDON COLOR

Parameters:

- COLOR: mandatory numeric grid color.

Examples:

GRIDON 14

Result and consequences: Enables the standard screen grid with the requested color.

## GRIDOFF

Availability: Local console and remote /o channel.

Accepted aliases: CGRIDOFF.

Meaning: Disables the diagnostic grid overlay.

General syntax:

GRIDOFF

Parameters:

- No parameters.

Examples:

GRIDOFF

Result and consequences: Disables the standard screen grid.

## PICTURE

Availability: Local console and remote /o channel.

Accepted aliases: CPICTURE.

Meaning: Saves a screenshot of the emulator screen.

General syntax:

PICTURE FILE

Parameters:

- FILE: output PNG, JPG or BMP filename.

Examples:

PICTURE zx81-screen.png

Result and consequences: A supported extension writes the current screen; unsupported formats fail.

## SETHOOK

Availability: Local console and remote /o channel.

Accepted aliases: CSETHOOK.

Meaning: Creates an execution hook.

General syntax:

SETHOOK ID TYPE [PARAMS...]

Parameters:

- ID: unique hook identifier.

- TYPE: hook kind; currently 0 stops on memory read/write.

- For TYPE 0, PARAMS are mandatory INITIAL and optional FINAL addresses.

Examples:

SETHOOK 1 0 $d000 $d3ff

Result and consequences: Creates an execution hook.

## REMOVEHOOK

Availability: Local console and remote /o channel.

Accepted aliases: CREMOVEHOOK.

Meaning: Removes a hook by identifier.

General syntax:

REMOVEHOOK ID

Parameters:

- ID: hook identifier.

Examples:

REMOVEHOOK 1

Result and consequences: If the hook does not exist, no state changes.

## HOOKS

Availability: Local console and remote /o channel.

Accepted aliases: CHOOKS.

Meaning: Lists active hooks.

General syntax:

HOOKS

Parameters:

- No parameters.

Examples:

HOOKS

Result and consequences: Lists active hooks.

## HOOKSHELP

Availability: Local console and remote /o channel.

Accepted aliases: CHOOKSHELP.

Meaning: Shows help for supported hook types.

General syntax:

HOOKSHELP

Parameters:

- No parameters.

Examples:

HOOKSHELP

Result and consequences: Shows help for supported hook types.

## CMOVEPARAMS

Availability: Local console and remote /o channel.

Accepted aliases: No public alias.

Meaning: Builds the framework's internal MoveParametersToAnswerCommand.

General syntax:

CMOVEPARAMS

Parameters:

- No parameters are executable under the current command contract.

Examples:

CMOVEPARAMS

Result and consequences: The command returns an empty answer when invoked directly; it is an internal response helper and is not advertised in the help file.

# 3.2 Inherited Sinclair Commands

## SYSVARS

Availability: Local console and remote /o channel.

Accepted aliases: CSYSVARS.

Meaning: Returns every system variable defined for the selected ZX80 or ZX81 ROM profile.

General syntax:

SYSVARS

Parameters:

- No parameters.

Examples:

SYSVARS

Result and consequences: Returns the current values through the system-variable definitions loaded for the selected ROM variant.

## SYSVAR

Availability: Local console and remote /o channel.

Accepted aliases: CSYSVAR.

Meaning: Returns one named system variable.

General syntax:

SYSVAR VNAME

Parameters:

- VNAME: mandatory variable name from the selected machine's system-variable table.

Examples:

SYSVAR D_FILE

SYSVAR E_LINE

Result and consequences: Returns the named variable and its current memory value; an unknown name returns an error entry.

# 3.3 ZX80/ZX81-Specific Commands

## ULA

Availability: Local console and remote /o channel.

Accepted aliases: CULA.

Meaning: Returns the current ULA and video-generation state.

General syntax:

ULA

Parameters:

- No parameters.

Examples:

ULA

Result and consequences: Returns the ULA state for the selected ZX80 or ZX81 ROM variant, including its current raster and display-generation data.

## ULAEVENTS

Availability: Local console and remote /o channel.

Accepted aliases: CULAEVENTS.

Meaning: Enables or disables visualization of significant CPU/ULA events.

General syntax:

ULAEVENTS ON|OFF

Parameters:

- ON: enables event visualization.

- OFF: disables it. The implementation accepts any single token; only the exact value ON enables the feature.

Examples:

ULAEVENTS ON

ULAEVENTS OFF

Result and consequences: Changes the ULA event-overlay flag used by the active ZX80/ZX81 screen implementation.

## DFDUMP

Availability: Local console and remote /o channel.

Accepted aliases: CDFDUMP.

Meaning: Returns a snapshot of the current display file.

General syntax:

DFDUMP

Parameters:

- No parameters.

Examples:

DFDUMP

Result and consequences: Returns the bytes selected by the active ZX80/ZX81 display-file mapping.

## CHARSDRAW

Availability: Local console and remote /o channel.

Accepted aliases: CCHARSDRAW.

Meaning: Returns textual drawings of all or selected character glyphs.

General syntax:

CHARSDRAW [0..63 ...]

Parameters:

- Character codes: optional values from 0 through 63; out-of-range values are ignored and duplicates are coalesced. With none, all 64 glyphs are rendered.

Examples:

CHARSDRAW

CHARSDRAW 0 1 32 63

Result and consequences: Returns textual glyph drawings from the character data visible to the selected ZX80/ZX81 model.

# 3.4 Local-Console-Only Commands

## LOADPRG

Availability: Local console only.

Accepted aliases: None.

Meaning: Loads, assembles and installs an assembler source through the emulator object.

General syntax:

LOADPRG FILE

Parameters:

- FILE: assembler source path.

Examples:

LOADPRG demo.asm

Result and consequences: Compilation or loading errors are reported by the local console.

## LOADBINARY

Availability: Local console only.

Accepted aliases: None.

Meaning: Loads a binary containing its destination address in the file header.

General syntax:

LOADBINARY FILE

Parameters:

- FILE: header-addressed binary path.

Examples:

LOADBINARY demo.bin

Result and consequences: Loads a binary containing its destination address in the file header.

## LOADBLOCKS

Availability: Local console only.

Accepted aliases: None.

Meaning: Loads a block-structured memory file.

General syntax:

LOADBLOCKS FILE

Parameters:

- FILE: block file path.

Examples:

LOADBLOCKS state.blocks

Result and consequences: Loads a block-structured memory file.

## DECOMPILE

Availability: Local console only.

Accepted aliases: None.

Meaning: Disassembles a memory region.

General syntax:

DECOMPILE ADDRESS BYTES

Parameters:

- ADDRESS: first address.

- BYTES: approximate byte count; output rounds to complete instructions.

Examples:

DECOMPILE $080d 64

Result and consequences: Disassembles a memory region.

## POSSIBLEPERS

Availability: Local console only.

Accepted aliases: None.

Meaning: Lists the peripheral IDs and construction attributes advertised by the active ZX80/ZX81 I/O peripheral builder.

General syntax:

POSSIBLEPERS

Parameters:

- No parameters.

Examples:

POSSIBLEPERS

Result and consequences: Lists IDs 10 (typewriter), 100 (cassette), 101 (cassette injection) and 102 (thermal printer). Cartridge ID 201 is accepted by the builder but is not included in this advertised list.

## CONNECTPER

Availability: Local console only.

Accepted aliases: None.

Meaning: Builds and connects a peripheral to the first compatible ZX80/ZX81 device.

General syntax:

CONNECTPER ID [ATTRS...]

Parameters:

- ID: peripheral-emulation ID (10, 100, 101, 102 or the additionally supported cartridge ID 201).

- ATTRS: optional construction attributes. For printer 102, use f:FILENAME and p:THERMAL or p:THERMAL-PS.

Examples:

CONNECTPER 100

CONNECTPER 102 f:printer.ps p:THERMAL-PS

Result and consequences: Connection succeeds only when the ID, attributes and a compatible target device are available; otherwise the answer reports an error.

## DISCONNECTPERS

Availability: Local console only.

Accepted aliases: None.

Meaning: Disconnects one or more peripherals.

General syntax:

DISCONNECTPERS ID1 [ID2 ...]

Parameters:

- IDn: connected peripheral IDs.

Examples:

DISCONNECTPERS 100 101

Result and consequences: Disconnects one or more peripherals.

## LOADPERDATA

Availability: Local console only.

Accepted aliases: None.

Meaning: Loads media or program data into a connected peripheral.

General syntax:

LOADPERDATA ID FILE [DEBUGFILE]

Parameters:

- ID: connected peripheral ID.

- FILE: supported data file; for example sampled TZX data for ID 100 or an O/P program for injection ID 101.

- DEBUGFILE: optional file that starts full-range format-related debugging after a successful load.

Examples:

LOADPERDATA 100 tape.tzx

LOADPERDATA 101 game.p load-trace.log

Result and consequences: The operation fails if the peripheral does not accept the supplied file type; otherwise the data is attached and optional debugging begins.

## EMPTYPERDATA

Availability: Local console only.

Accepted aliases: None.

Meaning: Creates and attaches empty media data when the peripheral implements that operation.

General syntax:

EMPTYPERDATA ID FILE

Parameters:

- ID: connected peripheral ID.

- FILE: destination whose extension selects a writable format.

Examples:

EMPTYPERDATA 100 blank.tzx

Result and consequences: The Sinclair cassette implementation can create an empty TZX container; unsupported peripherals or formats return an error.

## SAVEPERDATA

Availability: Local console only.

Accepted aliases: None.

Meaning: Saves a peripheral's current media data.

General syntax:

SAVEPERDATA ID [FILE]

Parameters:

- ID: connected peripheral ID.

- FILE: optional output path; it may be omitted when LOADPERDATA or EMPTYPERDATA established the current filename.

Examples:

SAVEPERDATA 100

SAVEPERDATA 100 copy.tzx

Result and consequences: Saves the current cassette data when the peripheral can retrieve it and the selected file format can be written.

## CLEARPERDATA

Availability: Local console only.

Accepted aliases: None.

Meaning: Clears all data currently loaded into peripherals.

General syntax:

CLEARPERDATA

Parameters:

- No parameters.

Examples:

CLEARPERDATA

Result and consequences: Commonly used to force a subsequent program/media reload.

Appendices

# Appendix A. Command Availability Summary

The table distinguishes the interactive console from the command channel of the non-console executable. Remote availability assumes ZX81Emulator.exe was started with /o and that the client is connected.

| Command family | ZX81EmulatorC local console | ZX81Emulator remote /o |
| --- | --- | --- |
| Standard builder | Yes | Yes |
| Inherited Sinclair builder | Yes | Yes |
| ZX80/ZX81 builder | Yes | Yes |
| LocalConsole emulator/media operations | Yes | No |

# Appendix B. Source-of-Truth Maintenance Rule

This file is a maintained repository artifact. Every change that adds, removes, renames or changes a ZX80/ZX81 startup option, a command-builder registration, a LocalConsole-only command, command syntax, parameter meaning, command consequences or user-visible formatted output must include a review and, when applicable, an update of docs/ZX81Data/ZX81Emulator_UserGuide.docx. The corresponding canonical .fmt source must be updated whenever the command's InfoStructure contract changes.

Audit the complete chain, not only the edited class: the ZX81Emulator entry point and ZX81::ZX81Emulator hierarchy for startup options; LocalConsole for local-only commands; and ZX81::CommandBuilder -> SINCLAIR::CommandBuilder -> MCHEmul::StandardCommandBuilder for builder commands.

# Illustrations

No se encuentran elementos de tabla de ilustraciones.NO ILLUSTRATIONS ARE INCLUDED IN THIS EDITION.

# Equations

No se encuentran elementos de tabla de ilustraciones.NO EQUATIONS ARE INCLUDED IN THIS EDITION.